# A TECHNICAL FEASIBILITY REVIEW, AND ECONOMIC VIABILITY ANALYSIS OF ALGAE BIODIESEL PRODUCTION IN THE UK

Michael Sinyangwe

Aston Business School

Aston University

MSc Business and Management

October 2010

Supervised by Dr. Prasanta Dey

Acknowledgements

I would like to show special thanks for the guidance given by Professor Prasanta Dey during this most interesting and exciting project.

I would also like to express appreciation to those who provided information and participated through their availability and involvement, so that this project could be completed.

Abstract

Over the past decade or so, a wide plethora of photobioreactors and lipid extraction methods have been designed and tested, along with forecasts of a wide range of biomass and subsequent biodiesel production rates. The optimisation of the technology to get highest possible yields at lowest possible costs should be left to scientists. However, the application of the technology to the real world is still lacking somewhat. This paper seeks to fill this gap by providing a techno-economic feasibility analysis of the major algae production system designs, as well as a comparative cost-benefit analysis between a 100,000 litre and 10,000,000 litre annual capacity configuration, of closed system, algae biodiesel plant. The Net Present Value method is utilised to determine the most economically viable design for investment, and uses a set of scenario forecasts to handle the uncertain future that lies within the energy domain. This research should therefore offer solutions to assess the viability of algae biodiesel projects, that could take advantage of the gap in the diesel supply market that is currently forming throughout the European Union. It fundamentally outlines the key areas of impact on algae ventures, that need to be taken into account when making decisions on project implementations, or investments in this domain. It has been found that at the current time, algae biodiesel is not competitive, but is very close to becoming so.

Contents

List of Figures and Tables	7

Introduction	Error! Bookmark not defined.

1.0	Context	8

1.1	The Overlying Issue	8

1.2	Problem Statement	11

1.3	Research Purpose	11

1.4	Objectives	12

1.4.1	Overview	12

General Literature Review	13

2.0	Introduction	13

2.1	Peak Oil and Fuel Prices	13

2.2	Biofuel Solution	14

2.3	What are Biofuels?	15

2.3.1	First Generation Biofuels	15

2.3.2	Second Generation Biofuels	16

2.3.3	Third Generation Biofuels	18

2.3.4	Fourth Generation Biofuels	18

2.4	Where Does Algae Fit In?	20

2.4.1	Why Algae Biofuels are Better than First and Second Generation Alternatives?	21

2.4.2	How Does Algae Produce Useful Lipid-Oils?	22

2.4.3	How is Biodiesel Produced from the Algae Lipid-Oils?	23

2.5	Potential Yield of Oil from Algae Grown Under Optimal Growth Conditions	25

2.6	Testing the Economic Viability of Algae	27

2.7	Algae Biofuel Production Challenges	28

2.7.1	Commercial Scale-Up of PBR Design	29

2.8	Why does Biodiesel Have the Advantage in the European Union?	31

2.8.1	Why has it Failed so Far?	32

2.9	Current European Union Movement in the Area of Biofuels and Vehicle Efficiency	34

2.10	Why is this Research Essential?	36

Methodology	37

3.0	Introduction to the Study’s Approach	37

3.1	Reasons for Choosing the Key Methodologies for Collection and Analysis of Data	38

3.2	Cost-Benefit Analysis	39

3.2.1	Net Present Value Framework	39

3.2.2	The Internal Rate of Return Alternative	40

3.2.3	AHP-based Example Study (Location Selection)	40

3.3	Summing Up the Methodology	40

Previous Feasibility Studies	41

4.0	Past Feasibility Studies Related to Algae Biodiesel Production	41

4.1	Methodologies for the Analysis of Economic Feasibility	41

4.2	Derived Major Economic Costs and Benefits of Production	42

Review of Photobioreactor Production Systems	43

5.0	Introduction	43

5.1	Technical Comparison of Algae Production Systems	43

5.2	Open Systems	44

5.2.1	Closed Ponds	45

5.3	Closed Systems	45

5.3.1	Flat-Plate Photobioreactors	45

5.3.2	Tubular Photobioreactors	46

5.3.3	Vertical-Column Photobioreactors	48

5.3.4	Oscillatory Flow Photobioreactor	49

5.4	So Where Should the Focus Transition Towards, and Why is it Focusing onto Closed?	50

5.5	Summary of Photobioreactor Designs	51

Building a Model to Ascertain the Economic Viability of Algae Biodiesel Projects	53

6.0	The Algae Biodiesel Plant Set-up	53

6.1	The Economic MODEL	55

6.2	Data Collection for the Economic Model	55

Total Cost For Algae Biodiesel Plants	56

7.0	Introduction	56

7.1	Adjusting for Oil Content	58

7.2	Total Production Cost	59

7.2.1	Algae Biomass Production Costs (Cost Pool 1)	59

7.2.2	Algae Oil Extraction Costs (Cost Pool 2)	59

7.2.3	Transesterification Costs (Cost Pool 3)	62

7.3	Byproduct Revenues Function	66

7.3.1	Total Annual Production Cost for Cost Pool 3	66

8.0	Project Assumptions	67

Diesel Price Scenarios	68

9.0	Diesel Forecast	68

9.1	Time Series Analysis	69

10.0	Building the Net Present Value Equation	72

10.1	Compensating for Investor Risk in the Discount Rate	72

10.1.1	How to calculate risk according to (Trevino, 2008).	72

10.1.2 Calculating risk premium	73

Discussion and Analysis of Future Strategy	74

11.0 Decision Based on the NPV Results	74

11.1	The 100,000L Algae Biodiesel Plant Configuration	74

11.2	The 10,000,000L Algae Biodiesel Plant Configuration	74

11.3	Cost Pool Analysis	75

12.0	Future Influences on Algae Biodiesel Success	77

12.1	Application of AHP-Based Location Selection Framework	77

12.2	Biodiesel Quality	82

12.3	UK Biodiesel Sales	83

12.4	Theoretical Contribution	84

12.5	Future Areas for Research	84

Conclusion	85

References	86

Appendices	93

# List of Figures and Tables

|  | Title | Page |
| --- | --- | --- |
| Table 1 | Cost and revenue components of algae biodiesel production | 42 |
| Table 2 | Example of Technical Options of Cultivation Equipment | 43 |
| Table 3 | Advantages and Disadvantages of Open and Closed Algal Cultivation Plants | 51 |
| Table 4 | Basic values of production for Open and Closed Cultivation Plants | 52 |
| Table 5 | Oil Content Percentages of a Selection of Major Algae Species | 58 |
| Table 6 | Algae Biomass Production Costs | 59 |
| Table 7 | Lists of Major Equipment Costs for Crude Esterified Algae Oil extraction | 59 |
| Table 8 | Lists of Total and Annual Fixed Capital Costs for Algae Oil Extraction | 60 |
| Table 9 | Lists of Direct Costs of Oil Extraction | 61 |
| Table 10 | Current Feasible Biodiesel Production Cost Efficiencies Depending on Capacity | 63 |
| Table 11 | Initial Costs of Biodiesel Reactor Capital | 64 |
| Table 12 | Variable Costs for Biodiesel Transesterification Process | 64 |
| Table 13 | Variable Cost per Litre of Biodiesel from Cost Pool 3 | 65 |
| Table 14 | Total Variable Cost per Litre of Biodiesel from Cost Pool 3 | 65 |
| Table 15 | Aggregate Revenue Function for Sale of By-Products | 66 |
| Table 16 | Total Annual Production Cost for the Transesterification of Algae Oil into Biodiesel | 66 |
| Table 17 | Project Assumptions | 67 |
| Table 18 | Historic Median UK Diesel Retail Prices | 70 |
| Table 19 | Forecasted UK Diesel Retail Prices | 71 |
| Table 20 | Scale of Relative Importance for Pair-wise Comparison | 78 |
| Table 21 | Number Codes for the Algae Biodiesel Specific Location Factors | 80 |
| Table 22 | Location Categorisation Guideline | 82 |

|  | Title | Page |
| --- | --- | --- |
| Figure 1 | Example of a Fourth Generation Biofuel | 19 |
| Figure 2 | European Union Trends in Diesel and Petrol Consumption (1990–2004) | 20 |
| Figure 3 | An Alga producing organic material | 22 |
| Figure 4 | The Transesterification Reaction | 23 |
| Figure 5 | Transesterification Input and Output Ratios | 24 |
| Figure 6 | Oil Yields for Various Feedstocks, Against their Apparent European Union Land Area Percentage Requirement | 26 |
| Figure 7 | Ishikawa Diagram Outlining the Key Barriers to Algae Biofuel Success | 28 |
| Figure 8 | Factors Affecting Commercial Scale-Up of PBR Systems | 30 |
| Figure 9 | Brazil 2010, Q1 Total FFV Production | 34 |
| Figure 10 | Share of Biofuels in Transport Fuel Consumption (EU 27 Countries) | 35 |
| Figure 11 | Methodology Approach | 37 |
| Figure 12a | Schematic of the Modules Comprising the Economic Viability Model | 38 |
| Figure 12b | Schematic of the Modules Comprising the Economic Viability Model | 55 |
| Figure 13 | Birds Eye Schematic of Open Raceway Pond | 44 |
| Figure 14 | Example of Real Open Raceway Pond | 44 |
| Figure 15 | Schematic for a Vertical Alveolar Panel Photobioreactor | 46 |
| Figure 16 | Schematic for an Inclined Horizontal Alveolar Panel Photobioreactor | 46 |
| Figure 17 | Schematic of Horizontal and Vertical Arrays of Tubular Photobioreactors | 47 |
| Figure 18 | Vertical Column Photobioreactor | 48 |
| Figure 19 | Internally Illuminated Photobioreactor | 49 |
| Figure 20 | Oscillatory Flow Photobioreactor | 50 |
| Figure 21 | Simplified Schematic of the Oscillatory Flow Photobioreactor | 50 |
| Figure 22 | The Cost Pools Associated with Algae Biodiesel Production | 53 |
| Figure 23 | Scale Factor Curve for Capital Cost | 63 |
| Figure 24 | Some of the Key Variables that Affect Crude Oil Prices | 68 |
| Figure 25 | Forecasted UK Diesel Retail Prices | 70 |
| Figure 26 | Diesel to Market Volatility Correlation, Used to Ascertain Beta | 73 |
| Figure 27 | Total Cost per Litre of Biodiesel | 75 |
| Figure 28 | Total Cost Percentage of each Cost Pool | 76 |
| Figure 29 | Hierarchical Structure | 79 |

# INTRODUCTION

# 1.0	Context

## 1.1	The Overlying Issue

There is increasing importance being placed on the provision of vital resources for modern societies. This is mainly because of predictions of significant expansion in world population over the next few decades. Current conservative estimates indicate world population by 2050 could

reach 8.9 billion (UN Department of Economic and Social Affairs, 2004). This means more resources will be needed, and the most severe stresses will be exerted on water, food, and energy supply. A significant amount of energy consumption arises because of transport purposes. In the UK this figure is 36% of total energy consumption (UK Department of Energy and Climate Change, 2009).

This sector is so important, because transport infrastructure enables growth. Road transport is key to world economies because business and social environments are becoming increasingly globalised and integrated. Internal EU freight-trade is especially reliant on the road haulage network. In the UK for example, trucks statistically deliver 85kg of goods to each citizen every day (see APPENDIX A for calculations). Vehicles also allow greater recreation opportunities.

The main source of energy for transport is crude oil. Increasing investment in oil extraction and production by the major oil companies has lead to an increase in oil available, but inevitably we are reaching a point where oil reserves are depleted and cannot be replaced with like for like discoveries, so it becomes harder and more expensive to extract the remaining oil. For example companies like Exxon Mobil and BP are pushing the boundaries of deep-sea oil extraction in the search for replacement oil reserves, which is one of the most expensive sources of conventional crude oil (Carroll, 2008). So we can see that diminishing returns are setting in, and causing inflated prices, squeezed margins, and ultimately eroding energy-profit. As a direct result, the production of oil is becoming less economically viable. We need an alternative source to eventually replace fossil fuels in transport. Many predictions indicate this will need to happen within the next couple of decades in order to avoid hampering growth in the global economy, as well as causing irreparable damage to our environment through greenhouse gas (GHG) pollution (Kavalov and Peteves, 2004).

There are two main options that have arisen out of this impending demand when coupled with societal appetite for renewable energy sources. These options are hydrogen or electric powered vehicles, both of which could theoretically eventually replace our fossil fuel dependence for transport (Kavalov and Peteves, 2004). It is difficult to decide which will be the most efficient, environmentally friendly, and economically viable option, due to the many independent variables and unpredictable future events that would influence the technologies. What can be said, is that they are long-term solutions. Although most of the technological aspects exist, it will take several decades to get suitable and efficient production, delivery, and storage infrastructure in place (Cherry, 2004). Until this happens, full mass market penetration cannot be achieved.

In the short-term we can conserve both fossil oil and the environment by improving conventional engine efficiency, however there is a limit to how efficient you can make an engine. We need a mid-term alternative to cover the gap between the production and demand for fossil fuel. Demand is increasing faster than it is currently possible to develop the infrastructure for electric and hydrogen transport (Cherry, 2004). Hybrids are another short-term facilitator, however their design is inherently inefficient due to always carrying around a heavy combustion engine at low speed. Even for full electric vehicles, they would currently shift the source of energy onto a much dirtier non-renewable energy source such as coal or nuclear power plants. One thing that seems apparent is that in the longer-term future, our road transportation energy ecosystem will likely evolve into a sophisticated and smart structure which is formed out of a mosaic of various renewable technologies as well as cleaner fossil fuel technologies. Each part of the mosaic will have its own optimum and most efficient transport purpose e.g. electric cars for short, intra-city trips whilst hydrogen fuel-cell cars for longer, inter-city trips.

Biofuels may be the perfect candidate to pick up the slack as they can by on large, be delivered and stored using current infrastructure, and most vehicles can use them safely without any need for modification at low blend percentages (Fontaras et al, 2010). It would be a simple case of gradually increasing blend percentages that are available and simultaneously incentivising car manufacturers to start releasing more flex-fuel vehicles (FFVs) onto the market. Several European car manufacturers sell flex-fuel vehicles that can run on a mixture of biodiesel and fossil diesel, in Brazil (Anfavea, 2010). Apart from the current costs per litre, a major obstacle to overcome for greater fleet penetration of biodiesel FFVs is the lack of availability of biodiesel blends at petrol stations in the UK. However, it is important to bear in mind that the source or type of biofuel greatly influences its real-term or life-cycle energy, cost, and environmental savings (Kavalov and Peteves, 2004).

## 1.2	Problem Statement

There are currently a number of studies that have looked at the costs, benefits, options, and timescales for the implementation of various biodiesel production facilities. These range from first generation, to a few even dealing with algae or third generation biodiesel. The problem is that many focus purely on the biomass production phase, or solely on the biodiesel production phase across a number of feedstocks. They do not focus on a full life cycle production project from algae through to biodiesel, and the only ones that do are more than six years out of date. A more up to date assessment is needed due to the devastating impact of the recession, as well as the generally ever quickening business environment that today’s world operates in.

On top of this, there has been no study found that tackles the issue directly from an investor viewpoint, with the common standard of net present value calculations. This is vital in understanding the distance that the technology has yet to travel to reach commercial standing, as well as bridging the gap between the scientific feasibility study that looks at the advantages and disadvantages of algae biodiesel, and the hard proof of concept that investors look for.

This study addresses both shortfalls.

## 1.3	Research Purpose

The aim of this dissertation is to evaluate the economic viability of implementing a commercial scale algae biodiesel plant, over the next 11 years, that can contribute to the UK’s fuel supply for ground-based transport.

The study explores different possibilities for the implementation of a production plant. It does not aim to develop an exhaustive technical analysis of the components and performance of the chosen algae biodiesel production system. Instead it develops a framework to ascertain the feasibility of developing an economically viable production plant that could be duplicated on a UK-wide basis if proved successful. The logical methodology is intended to help investors and authorities to make a decision about whether and when to invest in such an algae biodiesel project, as well as to highlight factors that should at the very least be taken into account. The model can essentially be modified for global application.

## 1.4	Objectives

1. To illustrate the suitability of increased algae biodiesel output in the EU.

2. To analytically critique past feasibility studies related to microalgae production projects.

3. To explain the feasible productive yields which major microalgae production systems can achieve.

4. To build an economic model that allows the determination of the economic attractiveness of considered algae production systems.

5. To analyse the future strategy for the most attractive project.

6. To verify the methodological framework that is created.

### 1.4.1	Overview

Objective 1 through 5 will require “desk research”, which accomplishes the evaluation of secondary scientific sources such as journal articles, research papers, and officially sanctioned data sets. The strengths of the secondary data assessed in desk research are many. Quick access to information via the internet, and the ability to see the method followed and therefore determine usefulness and validity of the information. This means more time available to search literature, design the research, and interpret the resulting data. The data is almost certain to be of equal or higher quality than any primary collection of data that could personally be achieved. The longitudinal nature of collected secondary data also allows the identification of key trends under temporally controlled variables (Bell and Bryman, 2007). However there are weaknesses in using secondary sources. These include a lack of familiarity with data, complex data making it difficult to decide which level of analysis should be employed, and no personal control over the data quality (Bryman and Bell, 2007). Objective 6 will require comparisons of results to other previous studies, as well as comments from an industry expert, so that the model’s application is appropriately justified.

A literature review will now take place in order to illustrate the suitability of increased algae biodiesel output for the EU, as well as to discuss the reasons for carrying out this research.

# General Literature Review

## 2.0	Introduction

The review in this section covers fuel price influences, and the problem that is created by volatility. It also covers explanations of the various types of biofuels, how they apply to the EU, and the productivities expected, as well as an overview of challenges still to be solved.

## 2.1	Peak Oil and Fuel Prices

We are coming out of an economic trough resulting from global recession, and oil prices are hovering around the $80-a-barrel range (BBC News, 2010). Just a few years ago, $30-a-barrel was considered a pretty high price for the market. This trend of increasing prices is due to the convergence of the “peak oil” concept with the laws of supply and demand. The real question about peak-oil, is how fast the world oil reserves will decline once past the production apex. Most importantly, how much can be extracted at a significant energy profit. Due to independent variables and unpredictable future events, the exact time-point when and indeed if biofuels will overtake crude oil can only be estimated. It is of vital concern to the biofuel debate because 97% of all energy consumed by transport vehicles remains carbon-based (Romm, 2006).

The “end of cheap oil” has already occurred, and the tightening constraint of reconciling energy supply with pollution, is starting to result in a situation where demand for energy cannot be satisfied. It is not hard to envisage the situation descending into increasing socio-economic disruption and international tension. Examining the forces and issues involved directly with peak-oil theory is beyond the scope of this research, however it forms the underlying root cause of our need to transition to transport renewables. The other key reason is for conservation of our global environment through pollution reductions. There is a significant disagreement on the size of our global oil reserve due to differences in the definition of conventional and unconventional oil (Greene et al., 2006). Our ability to derive transport fuels from unconventional oil such as shale-oil and oil-sands is saddled with difficulties, because current technology and infrastructure is not designed to take advantage of such fuel sources efficiently (Greene et al., 2006). It is also possible in some cases, that the exploitation of unconventional oil reserves will result in a negative net contribution to supply (Bentley, 2002).

## 2.2	Biofuel Solution

Biofuels could become a key resource to supply future demand for energy in the transport sector. They are classed as solid, liquid, or gaseous fuels, derived from renewable biomass sources (feedstock) (BBC, 2007). This includes both biogas and solid burning fuels, as well as liquid fuels. Biofuels may help fight climate change. They may help offset the dwindling oil and gas reserves. They may reduce dependency on unstable energy sources. They may create more jobs, and a more stable economy due to energy security. Despite this, clever innovations will be required to solve the list of problems associated with biofuels, and the large amounts of investment and research that will be needed in order to allow widespread use of the technology. At least in the transport industry, biofuels seem very promising. The transport sector relies upon liquid fuel due to its easy storage and combustion properties, and its relatively high energy density (BBC, 2007). In order to reflect this, the research will be based on liquid biofuel.

This study could help solve a variety of problems in our society as discussed above. It will be important, especially in the near future, due to the impending surge in transport energy demand, to mass market replacements for fossil fuels. By bringing the production and sale of transport energy in the form biofuel within national borders, the many inefficiencies of petroleum logistics may be solved, at the same time as providing more local jobs and curbing the damage we impose on our environment.

## 2.3	What are Biofuels?

Biofuels range from first generation such as corn-ethanol or palm-oil biodiesel, through to fourth generation where scientists genetically modify either the feedstock plant itself, or the organisms involved in its processing into a useable biofuel (Rubens, 2008). Fourth generation biofuels represent the newest and most groundbreaking technologies that require less fossil-fuel input to get the end-product biofuel, and generally produce less CO2 emissions over their life-cycle (Rubens, 2008).

### 2.3.1	First Generation Biofuels

First generation biofuels are derived from food crops. The major crops; corn, soy, palm, and sugarcane all have easily obtainable carbohydrates or oils. Conversion of these into biofeuls is straightforward. Carbohydrates are converted to sugars and then fermented into ethanol, whereas oils are transesterified and valuable FAME is separated out for use as biodiesel (Chisti, 2007).

First generation biofuels tend to need intensive fertilisers and pesticides, which ultimately causes environmental damage (Chisti, 2007). For example oceanic dead-zones are caused by eutrophication, where nitrogen-based fertilisers used on the corn crops eventually reach the sea. Microalgae thrive, and then oxygen levels deplete as they die and decompose, thus making it impossible for most other forms of life to survive. This brings wealth to farmers, but consequently makes it harder for fishermen relying on the coastal waters to make a living. The most noteworthy case is the dead zone emanating from the mouth of the Mississippi River, which has altered parts of the Louisiana coast so much that most sea life cannot survive (Malakoff, 1998).

First generation bioethanol is affected by fundamental problems. The carbohydrate source is fermented by yeast and generally requires expensive and environmentally unfriendly inputs (Schubert, 2006). Also, most vehicles cannot even run on 100% bioethanol in the European Union at the moment (ACEA, 2008b). Despite this, nearly all can handle a blend with fossil-fuel petrol following appropriate engine modifications (ACEA, 2008b). An example of such a blend is a flex-fuel such as E85 (85% bioethanol), but this currently experiences a narrow usage in the European Union (Kavalov and Peteves, 2004). Sugarcane derived ethanol is more efficient than corn derived ethanol, due to sugarcane’s higher growth yield (averaging 17 Mg ha−1) compared to corn’s (averaging just 3.7 Mg ha−1) (Kim and Dale, 2004). On top of this, sugarcane contains roughly 55% sugar per dry weight (Kim and Dale, 2004), whilst corn only contains about 4% sugar per dry weight (Kereliuka et al., 1995). Despite this, the usefulness of sugarcane is narrowed as it can only be cultivated in a relatively small area with specific climatic conditions (such as those in Brazil). The result is that the vast majority of American bioethanol is derived from corn. This situation has contributed to the US government creating measures of protectionism (Dufey, 2006). Such trade regulation of the biofuels industry means that bioethanol imports from Brazilian sugarcane are uneconomical for use in the USA, however there is greater promise for it in the European Union.

Another source for first generation biofuel is yellow grease. It is much cheaper than plant oil derived biodiesel, but issues do exist (Canakci and Gerpen, 2001). There will always be a constrained level of demand for restaurant food and so only a certain amount of oil will ever be used to cook food. This means that there is a limited supply, and so it can only make a limited contribution to biofuel supply.

Two key problems that first generation biofuels carry is that all tend to result in more carbon emissions over their life-cycle than the equivalent fossil-fuel. In some cases, production alone requires as much, if not more fossil-fuel than is gained in biofuel, so there is a net loss of fuel (Kavalov and Peteves, 2004). On top of this, fuel production competes directly with food production, which means that plant products and land are diverted from human consumption (Schubert, 2006). This evidently leads to food price inflation. However it is important to realise that food price rises could have various other influencers. For example, rising energy costs due to lack of oil supply increasing farming costs, coupled with increasing demand especially in developing countries such as China. Trade restrictions (protectionism), and greater demand for meat and dairy from developing countries also contributes. These issues mean most research and development projects are not pursuing first generation biofuels.

### 2.3.2	Second Generation Biofuels

Second generation biofuels use non-food biomass as feedstock, but more specifically the lignocelluloses in such materials (Elnashie and Garhyan, 2004). The feedstock can be a dedicated crop like switchgrass, or it can be agricultural or industrial waste similar to corn stalks and cobs, or woodchips. Carefully designed processes involving chemicals and microbes turn the tough cellulose into sugars, and ferment them into ethanol. Alternatively, a technique known as thermochemical gasification can be used to produce ethanol (Kim and Dale, 2004). Researchers and developers aim to develop an automated, cost-efficient process for producing alcohols. The enzymatic cellulolysis still requires around 40 to 100 times more enzyme per gallon of ethanol (second generation biofuel production) than for the enzymatic hydrolysis of corn starch (first generation biofuel production) (Merino and Cherry, 2007). Designing more efficient enzymatic or catalytic processes must be a priority.

A key issue with second generation biofuel is that it requires the removal of biomass that would otherwise degrade and replenish the soil quality. This negative effect will only be apparent after a few years, by which time soil humus is reduced and therefore diminishes the productive capacity of the land (Hopkin, 2008). Fertilisers are the usual remedy, but this creates other environmental issues. For example, eutrophication causing dead-zones in the oceans, rivers, and lakes (Malakoff, 1998). Regulation and good practice should be administered in order to avoid such a situation. On top of these issues, ensuring that less carbon is emitted during the lifecycle production of the applied fertilisers, compared to the carbon absorbed by the crops that grow on them is necessary. The profit gained from any increase in yield compared to the cost of using the fertiliser must also be realisable to warrant their use.

If cultivating LIHD (Low-Input-High-Diversity) grasses as feedstock, it is rational to expect that more carbon is left in the soil (in the form of roots), than is released over the entire biofuel lifecycle. This makes them a carbon-negative biofuel feedstock. Research indicates that high-diversity grasslands have bioenergy yields that are 238% greater than monoculture yields, over a ten year period (Kim and Dale, 2004). These researchers also found that net carbon dioxide sequestration of LIHD grasslands, was around 4.4 Mg hectare–1 year–1, which exceeds carbon dioxide released during subsequent biofuel production (0.32 Mg hectare–1 year–1) (Tilman et al., 2006). There are advantages, especially when using hardy grasses such as miscanthus or switchgrass. They are easy to maintain, fast growing, do not require fertilisers, and do not need much water. As a result, they can be grown on marginalised land, which reduces issues of fuel versus food (Tilman et al., 2006).

### 2.3.3	Third Generation Biofuels

These come from algal lipids (oils) that can be easily converted into a range of biofuels. The remaining dry-biomass residue can be combusted to generate heat, anaerobically digested to produce methane, used as an ethanol feedstock, used as fertiliser, or fed to animals (Edwards, 2008). It is a multipurpose crop plant that is highly adaptable. They can grow on any land type (even deserts) (Edwards, 2008). This removes competition with agricultural land and food supplies. Algae can also be grown with sea or waste water, which means that fresh water can be conserved for essential areas such as food crops and human use (Edwards, 2008). This advantage will become more important as our earth looses fresh water reserves due to global warming and excessive crop irrigation.

On top of this, the bi-product oils of algae offer profit boosting opportunities, because they are pure enough to be used in the nutraceutical and pharmaceutical industries (Edwards, 2008). The oil produced from algae contains no sulphur, which means that combustion emissions do not cause formation of acid rain clouds in the atmosphere (Edwards, 2008). It also removes an expensive de-sulphuring stage required as part of the downstream processing of fossil petrol and diesel (PALL Fuels and Chemicals, 2005). This lack of sulphur also reduces damage to catalytic converters of car exhausts. This subsequently allows greater efficiency in converting toxic CO and NO to CO2 and NO2.

### 2.3.4	Fourth Generation Biofuels

Fourth generation biofuel technologies aim to genetically optimise feedstocks so that they capture higher levels of CO2. These feedstocks are then efficiently broken down and processed into biofuels by genetically modified microbes or by chemicals. Modification of CO2 fixation mechanisms in plants, as well as genetic modification of processing organisms of the biofuel production phase is crucial because it allows fourth generation biofuels to be a carbon negative source of fuel (Rubens, 2008). An example is shown in Figure 1, over page.

Figure 1: Example of a Fourth Generation Biofuel Strategy

Source: Rubens, 2008

## 2.4	Where Does Algae Fit In?

They are classed as feedstock for third generation biofuels. Algae fossils date back to 3.5 billion years ago (Whitton and Potts, 2000). At this time there was no oil. Over time, algae turned into oil. Now, algae are still around whereas oil is rapidly depleting. In the future, oil will run out whereas algae will not. It seems like algae may be the perfect choice for future fuel needs. Some algae can double their mass several times per day, more than half the mass in the form of lipids or triglycerides (fatty oils) (Edwards, 2008). They have higher oil yields per unit area than any other biofuel feed stock, as has been proven in figure 6 later in this review. These oils can be harvested and converted to biodiesel, but also jet fuel and many others (Edwards, 2008). With diesel consumption on the rise (see Figure 2), and more people opting for diesel cars (Kavalov and Peteves, 2004), algae biodiesel is poised to get an even greater market share.

Figure 2: European Union Trends in Diesel and Petrol Consumption (1990–2004)

Source: Pickett et al., 2008

Past studies, for example the US department of energy’s aquatic species programme, assessed the cost of growing algae and found that at the time (1978) it was not competitive (Sheehan et al., 1998). However due to increases in oil price, improvements in the efficiency of algae biofuel technology, and the realisation that other generations of biofuel technology are far less efficient, it is almost competitive and will become even more so in the future. The study focused on open pond growth systems that use saltwater or wastewater. Problems arose due to algae exposure to contaminants and other organisms brought in by air. This causes a lower efficiency because the useful algae have to divert energy resources and time to fend off, and out-compete the intruding species, or to nullify toxic contaminants (Sheehan et al., 1998). Comparatively, the closed bioreactor system allows the algae to use all available energy for growth.

### 2.4.1	Why Algae Biofuels are Better than First and Second Generation Alternatives?

The key difference lies in the type of feedstock organism involved. First and second generation biofuels rely on land based macro-plants, whereas algae are a water-based micro-plant (Edwards, 2008). This difference gives algae the advantage in a number of ways:

• Structure: Land plants waste a lot of energy to build lingo-cellulosic support structures such as roots, stems, and leaves so that they can optimize their orientation for photosynthesis whilst being resilient to wind and other impact forces. Algae get their support from the water they thrive in (in the form of osmotic pressure), so more energy is used to produce useful products such as carbohydrates and lipids (Edwards, 2008).

• Speed: Land plants have long growing seasons, and so it often takes more than 3 or 4 months to produce a useful crop. Algae have an exponential growth rate so fast that each cell can produce over a million offspring in a matter of hours. This means the lag time between photobioreactor set up and actually producing biodiesel can be very short (Edwards, 2008).

• Direction: Land plants grow slowly in one general direction, taking weeks to double their biomass. Algae grow in all directions simultaneously (360°), which allows them to triple biomass daily under optimal conditions (Edwards, 2008).

• Reliable production: Unpredictable events such as drought, disease, and bad weather can devastate any land-based crop. The closed photobioreactor system effectively allows you to control any of these variables for algae growth. In addition, algae have evolved a fast growth switch which means they slow down their growth rate quickly when conditions are bad in order to conserve energy e.g. low light, and will immediately continue their explosive growth once optimal conditions are returned. This gives a very precise and responsive control of when and how much biodiesel can be produced (Edwards, 2008).

• Composition: Land plants tend to have high levels of cellulosic material, which diverts energy away from oil production. On the other hand, certain algae strains can naturally produce more than 50% of their biomass as lipid-oils that can be converted directly to biodiesel (Edwards, 2008).

### 2.4.2	How Does Algae Produce Useful Lipid-Oils?

Algae require four key inputs in order to grow and produce oil (see Figure 3). These include water and light which are plentiful and so are not significant barriers to development of algaculture technology (Edwards, 2008). The other two inputs can have a big impact on the cost of producing algae oil. These are CO2 and nutrients, both of which must be cost minimised in order to get a decent cost to productivity ratio that can compete with fossil fuels (Edwards, 2008). However, acquiring CO2 may quickly become inexpensive due to emissions tax increases (Abadie and Chamorro, 2008).

Figure 3: An Alga producing organic material (The glucose undergoes a series of biological reactions that transform it into lipids.)

Photosynthesis

C6H12O6 + 6O2

6H2O + 6CO2

Respiration

Light

Water + Carbon dioxide

Glucose + Oxygen

Nutrients

Algae Cell

Source: Falkowski and Raven, 2007

Algae absorb carbon dioxide, and convert it to simple sugars, proteins, lipids, and waste oxygen. This all happens in a process that requires light, called photosynthesis (Falkowski and Raven, 2007). This means that there is a constant demand for carbon dioxide for growth, as well as a constant need to get rid of oxygen as it builds up within the closed photobioreactor system (oxygen is toxic for algae above certain concentrations) (Ghirardi et al., 1997). Water stores little dissolved carbon dioxide naturally, so algae need CO2 to be pumped into the water constantly (Edwards, 2008). As a result, the photobioreactor will need a cheap and plentiful source of carbon dioxide. Nutrients will be another input that must be cost minimised. The challenge for algae cultivation is in ensuring that nutrients are constantly available, and in the right proportions for algal growth.

### 2.4.3	How is Biodiesel Produced from the Algae Lipid-Oils?

Algae lipid-oils are made up of three esters attached to a glycerol molecule, forming a triglyceride (Chisti, 2007). An ester is a hydrocarbon chain just like the hydrocarbon chains that comprise both diesel and petrol (Chisti, 2007). Biodiesel is produced through a process known as transesterification, which separates glycerol from the fatty acids in algae oil. The esters then attach to and react with alcohol molecules (e.g. ethanol) to form biodiesel (Chisti, 2007). This chemical process is catalysed by sodium hydroxide or potassium hydroxide traditionally, but more efficient and environmentally friendly new methods use solid base catalyst beds as they are reusable and more environmentally friendly (Chisti, 2007). See figure 4 below for an idea of the reaction that takes place.

Figure 4: The Transesterification Reaction

Source: Chisti, 2007

The waste glycerol can be used to make other products e.g. soap (Chisti, 2007). The product fatty acid methyl ester (FAME) is a biodiesel that can be used as a pure fuel (B100), or it can be blended with fossil diesel (Chisti, 2007). Neither require any engine modification, although to prolong function and durability, each engine will have a certain blend compatibility range designated by the manufacturer (ACEA, 2008b). Below is figure 5, which illustrates the ratio of inputs and outputs of the typical transesterification reaction.

Figure 5: Transesterification Input and Output Ratios

| Process Inputs | Volume Percentage |  | Process Outputs | Volume Percentage |
| --- | --- | --- | --- | --- |
| Vegetable Oil | 87% |  | Biodiesel | 86% |
| Alcohol | 12% |  | Alcohol | 4% |
| Catalyst | 1% |  | Fertiliser | 1% |
|  |  |  | Glycerin | 9% |

Source: Methanol Institute, 2006

## 2.5	Potential Yield of Oil from Algae Grown Under Optimal Growth Conditions

Figure 6 below shows that commercial algae ventures could produce more than 100,000 litres of algae oil per hectare per year. This would allow algae to totally outcompete all land based alternative feedstock sources for vegetable oil that can be readily turned into biodiesel. First and second generation feedstocks use fertile cropland and large amounts of water, and from Figure 6 below, we can see that they actually provide a dismally low yield when put into context with algae. Also it is not feasible to grow the high yielding land plants such as the Oil Palm and Coconut trees in the European Union, due to climate differences (Edwards, 2008).

According to Figure 6, replacing total transport fuel consumed in the European Union with microalgae derived biodiesel would require at least 370 million tonnes of oil equivalent biodiesel annually, under the current consumption rate (see APPENDIX B). Pursuing any other biodiesel crop, whilst meeting existing European Union transport fuel needs, would require unsustainably large cultivation areas. This is also demonstrated in APPENDIX B. Here, the average oil yield per hectare from various crops (column 2), is used to calculate the land area needed to meet 100% of European Union transport fuel needs (column 3), and this allows calculation of the land area needed expressed as a percentage of the total suitable land area of the European Union (e.g. non-mountainous areas). The data suggests that 1-2% of the total European Union suitable land area is sufficient to produce algal biomass that satisfies 100% of the transport fuel needs, however the aim is merely for algae to contribute a significant portion of energy to the transport mix. In light of this, it is likely that between 0.3% and 0.7% of such land could satisfy a third of total transport energy consumption in the European Union. The microalgal oil yields in APPENDIX B are calculated from experimentally demonstrated biomass productivity in photobioreactors, and so it is important to bear in mind that once scale-up of production has occurred, the actual yields may be somewhat different. In addition to this, realisable biodiesel yield per hectare from the parent crop oil has been estimated at roughly 80% (Chisti, 2007).

On the next page you can see the comparison between the main potential biodiesel feedstocks.

Figure 6: Oil Yields for Various Feedstocks, Against their Apparent European Union Land Area Percentage Requirement

Source: see APPENDIX B for raw data and calculations

In light of figure 6 above, microalgae emerge as the only feasible biodiesel source that can comfortably replace fossil diesel for the European Union.

## 2.6	Testing the Economic Viability of Algae

Algae technology is not yet mature, and as such a dominant design still eludes the industry (Edwards, 2008). Investors therefore have to consider carefully the most financially beneficial projects to fund. Fossil fuel oil price slumps make consumers and investors lose interest in renewables, however different investors have personal expectations about risk, return on investment, and length of investment (Liansheng, 2000).

The high up-front investment of algae biodiesel facilities is a major obstacle to the success of this technology, which makes external investors almost a necessity (Edwards, 2008). Only a few studies on the economic viability exist, and they make extensive use of assumptions due to the lack of in depth raw data (Kavalov and Peteves, 2004). Nearly all highlight that capital, labor, operational, and maintenance costs are key barriers to making the technology economically viable at large scales, and so it can be cost-competitive with fossil fuel (Brown, 2009 and Dimitrov, 2007).

## 2.7	Algae Biofuel Production Challenges

Commercial scaling of the technology is the primary production challenge (Barbosa, 2003). Lab prototypes enable much greater algae productivity than has been realised in field testing (Barbosa, 2003). Key areas for improvement, leading to successful commercialisation are also shown in Figure 7. They include growing systems, processing technology, inputs, marketing, and scaling.

Figure 7: Ishikawa Diagram Outlining the Key Barriers to Algae Biofuel Success

Sources of Information: Edwards, 2008 and Barbosa, 2003 and Chisti, 2007

Growing systems are an important remaining hurdle, because of the already highlighted multitude of designs that are possible. This quandary also forms relationships with the algae strain selected for cultivation, because the design of the photobioreactor is usually tailored to fit with the species of choice. As will be shown later in this study, the costs arising from maintenance are large and must be tackled urgently.

Marketing is probably the weakest link because algae biofuel has enjoyed relatively little exposure to the wider public, and any such interaction would likely tar the algae image with the same negative connotations associated with first generation biofuels. For this reason it would be necessary to make sure that the distinction is clear between the generations. Use of the green consumer advocate as a nexus to launch the concept of algae biodiesel would likely increase the impact and efficacy across the general public. Considerations of the use of all bi-products from algae production need to also be addressed, as a zero waste philosophy is the best way to make a product successful and economical.

Inputs are another barrier to success because they are variable, and especially the nutrient source because it is seen as the most costly of the input costs (Edwards, 2008).

Process development and optimisation will be an instrumental area of the cultivation technology that drives algae biofuels. The ability to automate much of the process is essential in lowering costs to the required levels for commercial feasibility (Edwards, 2008).

Finally the Scale-Up barrier can be split down into four key components as follows in section 2.7.1.

### 2.7.1	Commercial Scale-Up of PBR Design

Yield comparison between different systems is merely theoretical because no large commercial facilities using photobioreactors exist at present (Barbosa, 2003). However, it is useful to consider the potential of systems in order to calculate optimal capacities and associated land requirement that will allow a business venture to meet expected demand.

Reactor design is followed by optimisation, with the chosen system ideally customised to the strain of algae being grown. The most critical scale-up optimisation issues to be addressed are light distribution, mass transfer, hydrodynamic stress from mixing characteristics (rate of flow, and turbulence), and circulation time (Barbosa, 2003). This is illustrated on Figure 8 below.

Figure 8: Factors Affecting Commercial Scale-Up of PBR Systems

Source: Author

## 2.8	Why does Biodiesel Have the Advantage in the European Union?

Mainly because Europeans are so fond of diesel engines. According to the ACEA more than half (53%) of new cars in the European Union are sold with a diesel engine. In other continents the diesel market is far less developed (ACEA European Automobile Manufacturers’ Association, 2010).

Diesel engines used to be smelly, noisy, and produce high amounts of particulate emissions that damage people’s health. However due to strict European Union emissions standards, policy makers have promoted the development of high-technology diesel fuelling systems, new combustion processes, and after-treatment systems in order to achieve lower particulate emissions e.g. NOx reductions (ACEA European Automobile Manufacturers’ Association, 2010). Diesel engines are also the lowest emitters of CO2 per energy output, exhibit very high fuel efficiency, and benefit from low fuel prices because of diesel’s lower fuel tax in the European Union (Vierhout, 2010). The taxes were intended to decrease costs for the economically important trucking sector in the EU, however the fuelling logistics between trucks and passenger cars are not separated (Vierhout, 2010). This has allowed the consumer to benefit from the low taxation on diesel. As a result of high demand for diesel passenger vehicles, manufacturers invested in production of more fuel efficient diesel engines, with priority above petrol engines (Vierhout, 2010). Even in the UK where diesel prices are slightly higher than petrol, the diesel car is still dominant (ACEA, 2009). The European Union has a surplus of petrol refining capacity, exporting a lot of it to the USA (ACEA European Automobile Manufacturers’ Association, 2010). We simultaneously experience diesel deficit because of the high percentage of diesel vehicles. The consequence is that there are many more diesel vehicles in the European Union (ACEA European Automobile Manufacturers’ Association, 2010). This means we currently import many equivalent barrels of diesel from outside the European Union (mainly from Russia) (Vierhout, 2010). In light of this, we should promote biodiesel to fill the fuel supply gap, rather than promoting ethanol to unnecessarily enlarge our petrol supply.

Added together, this all means that countries in the European Union are ideally placed in terms of favourable consumer tastes for diesel as well as technical experience in diesel engines, and world-leading technological ability to push through a highly beneficial transition to high biodiesel blends.

### 2.8.1	Why has it Failed so Far?

If you look at the bigger picture of energy and transport throughout the last 200 years, biofuels are still performing much better in terms of time to mass market compared to others. For example, conventional oil and petrol took around 100 years to penetrate mass market. The first oil was drilled at the start of the 1800’s, yet was only widely used in the first mass produced cars e.g. Ford Model T, in 1908 (Dickey, 1959 and Clymer, 1955). It is a smaller struggle under current conditions, to get algae technology and infrastructure to a level where it too can serve the mass market due to technological advances, and algae derived biodiesel has only been pursued since the mid 1970’s market (Edwards, 2008).

There is a key underlying reason why the biofuels market has not developed significantly in the European Union. This is predominantly because of the ambiguous signals that the energy and vehicle manufacturing industries receive. There has been weak commitment to setting higher biofuel blend requirements for fuel suppliers through EU regulations (Vierhout, 2010). On top of this, the whole energy industry is currently in a period of “drift” because of uncertain European Union carbon taxes, carbon allowance market price volatility, and changing efficiency standards. This means that investing in clean energy technologies is very risky. This immediately rules out many venture capitalists, and significantly reduces the availability of funding for development of the necessary technology and infrastructure that algae and biofuels in general require (Clean Tech Group LLC, 2009). However this is likely to change in the near future as agreements are currently being drawn up by the EU (ACEA, 2008a).

Proposed European rules on measuring car emissions could pose another serious threat. The new standards dictate that fuel efficiency should be the focal point for reducing emissions, but clauses that part of this can be realised through biofuel compatibility (European Parliament and EU Council Legislation, 2010). This would significantly benefit vehicle manufacturers through tax savings if higher blends of biofuel were widely available. The issue is that large oil producers who own the majority of UK petrol stations have no incentive to make high biofuel blends available at their petrol station pumps. The resulting EU stance is that FFVs cannot be classed under this initiative (Vierhout, 2010). They fear that otherwise FFV production would increase rapidly but have no impact on emissions because the biofuels would not be available. The FFVs would simply be using highly polluting petrol or diesel (Vierhout, 2010). The European Union rules have effectively built an unhelpful environment for fast development of the biofuels industry. No FFV production correlates to no demand or supply of biofuel, and no high biofuel blends at the pump means no point in producing FFVs. If the European Union reversed their opposition and therefore offered FFV tax breaks, then FFV production would increase (Vierhout, 2010). The vehicle manufacturers would almost certainly demand that petrol stations increase the availability of biofuel at the pumps. Big oil companies could then have an understanding with vehicle manufacturers to simultaneously improve conventional vehicles. This would allow conventional engines to compete with FFVs more equally in terms of efficiency and pollution and so consumers will still want to use their fossil fuel resources. The outcome is a win-win situation where more biofuel, more FFVs, and better fuel efficiency in conventional vehicles are all achieved simultaneously.

We know that such governmental support for FFVs will be decisive for the progression of algae biofuel because of the Brazilian example. In Brazil, the government has put its weight behind the biofuel industry for many decades now. The result is that more than 50% of their transport fuel is biofuel such as sugarcane derived ethanol (Global Bio-Energy Partnership, 2007). Even the vehicle manufacturers are benefiting dramatically. The domestic demand for FFVs in Brazil allowed Brazilian companies to pass the milestone of producing their 10 millionth aggregate FFV on the 4th March 2010 (Green Car Congress, 2010). Many do not even produce cars that run on pure fossil fuel anymore, and since all the major vehicle manufacturers in Brazil have a global presence (see figure 9, over page), this could be a valuable stage to rollout FFVs to the rest of the world. A well developed global FFV market would encourage greater production and availability of biofuels (especially ethanol and biodiesel), as results in Brazil have shown for decades.

Figure 9: Brazil 2010, Q1 Total FFV Production

Source: Anfavea, 2010

## 2.9	Current European Union Movement in the Area of Biofuels and Vehicle Efficiency

The European Union Renewable Energy Directive (RED, Dir 2009/28/EC) forces 10 percent renewable energy in transport by 2020 (ACEA, 2008a). Many strategies are available to achieve this, all of which require biofuels to play a major role (European Council for Automotive R&D, 2009). In this same report they admit that with strengthening regulations, conventional diesel engines will become more expensive, less efficient and will encounter difficulties in reaching future emission standards. This will have consequences on the possibility for Europe to fulfill its C02 objectives. The easiest and most effective way around this, would be to raise the biofuel blend percentages at petrol stations. This is currently being explored under a Fuel directive, which has not yet addressed the problem of European Union biofuel synchronisation (ACEA, 2008b). So available blends as well as quality of fuel must be synchronised across the European Union supply network.

Today’s European Union fuel blend standards (EN228 and EN590) permit up to 5% ethanol in petrol and 5% FAME (Fatty Acid Methyl Ester) in diesel (ACEA, 2008b). Despite this, most fuel producers still supply fuel blends of well below 5% biofuel (ACEA, 2008a). There is a lot of scope to make better use of biofuels, especially now that the European Union Commission’s Renewable Energy Strategy sets out the biofuel blend targets for post-2010 at 10% ethanol (E10) and 7% for biodiesel (B7), pending agreement on the Fuel Quality Directive (ACEA, 2008a). All vehicles produced from 2010 onwards will be fully compatible with these new standards (ACEA, 2009). This is due to a universal agreement between all members of the ACEA to create this benchmark. Such vehicles are designed to run on higher blends of biofuel.

In order to preserve the function and durability of engines, the aspect of quality becomes a concern. This means good quality of the biofuel itself, as well as precise and accurate measuring when blended into fossil fuel (ACEA, 2009). The older vehicles that can run on a maximum of 5% biofuel blend would also have to be accommodated at petrol stations until 2016 (ACEA, 2008b). Future events hold promise of continued incremental increases of biofuel blend percentages in line with our ability and infrastructure to distribute them. In 2015 the EU will consider moving to low/medium blend biofuels (ACEA, 2008a). Below is figure 10, showing the current exponential trend in biofuel consumption in the EU, for transport purposes. From this it becomes clear just how quickly biofuels are becoming an integral part of our energy supply.

Figure 10: Share of Biofuels in Transport Fuel Consumption (EU 27 Countries)

Source: Eurostat, 2010a

## 2.10	Why is this Research Essential?

Summing up the preceding information, it is obvious that biodiesel is an integral part of all future EU plans to meet energy supply for transport. We are entering an era of increasingly constrained fossil resources. Biodiesel alternatives are the most promising in terms of environmental, economic, and social aspects. Crucially, biodiesel demand is latent, which shows that in order to roll-out this technology one must expect supply to stimulate demand. This balance can be equalized if FFV manufacturers were further incentivised to increase production simultaneously with more support from EU regulations. In light of this, a study of the potential biodiesel market, as well as an economic feasibility analysis for the implementation of an algae biodiesel production project is ideal. It will aid future decisions of investment and business timing of algae biodiesel ventures. EU efforts to increase biodiesel production by helping biodiesel producers to develop profitable enterprises through regulatory change, will be vital for any business success in this industry.

# Methodology

## 3.0	Introduction to the Study’s Approach

The methodology is comprised of four key stages. The first stage is to assess past feasibility study methodologies related to algae biodiesel in order to select useful ideas for this feasibility study. The second stage is to elucidate the technical properties of the various algae cultivation systems that are seeing success in the industry. The third stage is to build the economic viability model upon which investment decisions can be made, so that the optimum plant technology, size, configuration, and project can be selected for future implementation. The final stage is to analyse the future strategy of the chosen project. The framework for this methodology is fulfilled in a logical fashion, as depicted in figure 11 below.

Figure 11: Methodology Approach

STAGE D

STAGE C 1

STAGE B 1

STAGE A 1

Assess Previous Feasibility Studies

Explain Technical Production Options

Build Economic Viability Model

Analyse Future Strategy

See Figure 12a over page, for a preliminary foresight of the modules that will constitute the economic viability model for Stage C of the methodology.

Figure 12a: Schematic of the Modules Comprising the Economic Viability Model

Source: Author

The overall purpose of the economic model that has been developed, is to construct a simulation of the future biodiesel environment that allows you to assess the profitability associated with algae biodiesel production systems, and therefore provides a basis upon which investment can be made, by taking into account the nature of uncertainty involved. The intricacies of the above model are appropriately described later on in this dissertation.

## 3.1	Reasons for Choosing the Key Methodologies for Collection and Analysis of Data

This methodology will be the most effective in terms of collecting the data needed to answer the original research question. It follows a mixed-method, as it uses a lattice of both aspects of the qualitative-quantitative paradigm. This means the various methodological strengths can be capitalised on, and the weaknesses offset somewhat. The two methods are mutually illuminating, because without a quantitative economic analysis, there is no substance upon which definitive decisions can be made. Likewise, without the qualitative aspects of stages a, c, and d, the economic viability model cannot be held credible or representative to an extent that correlates with reality. The two research strategies in this case can be classed as complementary (Hammersley, 1996).

## 3.2	Cost-Benefit Analysis

In general terms, a cost-benefit analysis involves making an investment decision that will maximise the present value of all benefits less that of all costs, subject to specified constraints (Brent, 1996). In this case, only the private benefits and costs that can be financially assessed are included; the necessary data is derived from market prices; the market rate of interest is to be used for the discounting of annual cashflows; and, the main constraint is that of the initial investment fund. In order to avoid bias, all projects will be assessed equally against the same set of factors.

Due to the inherent nature of investment needed to set-up a commercial algae biodiesel venture, it is necessary to take account of two major forces. To an investor, these forces come in the form of time and risk. They form two basic principles that investors use to make decisions, whereby money is worth more now than it is in the future; and, that on aggregate, a safe investment tends to be better than a risky investment. In order to compensate for these fundamental rules, the cost-benefit analysis will be made in the form of Net Present Value (Trevino, 2008).

### 3.2.1	Net Present Value Framework

Zero NPV projects repay their original investment plus the investor rate of return. Positive NPV indicates a better return, whilst negative NPV indicates a worse return (Trevino, 2008). NPV analysis assumes a project requires an initial investment of C0 at time 0, and at annual time intervals it generates a certain cashflow based on a number of determinants, from time 1 to time t. In order to find the NPV of the project, the sum of an extended stream of cashflows must be calculated, using the discounted cashflow formula (Trevino, 2008). The formula for NPV therefore is as follows:

NPV=-C0+C1(1+r1)1+C2(1+r2)2+C3(1+r3)3+…+Ct(1+rt)t

In this analysis a total production cost function has been calculated. As a result, the only necessary numerator components are the initial fixed capital investment, and annual variable and fixed costs. Usually the determinants are levelised i.e. they stay constant throughout the analysed time period, however in this case determinants will vary based upon manipulations of collected data i.e. the projections for the key saleable product, biodiesel. Another important point is that in order to avoid bias, all algae biodiesel plants must be subject to the exact same input conditions. For this reason, any data that doesn’t apply to all of them was not factored in and was deemed negligible.

### 3.2.2	The Internal Rate of Return Alternative

This is the other main discounted cashflow approach for investment decisions (Trevino, 2008). The difference compared to the NPV however is that the rate of return looks at each project in isolation from the external environment and as such does not account, as an example, for inflation (Trevino, 2008). This is deemed unsuitable for the purpose of this study as the environmental factors of the biodiesel business environment likely have a large bearing on the profitability of any venture within.

### 3.2.3	AHP-based Example Study (Location Selection)

An example study of the location selection approach has been demonstrated within the analysis of future strategy. Identified algae biodiesel specific location factors were put through the proposed location selection framework process. The purpose of the example study is to ascertain the optimum plant location in the UK, as well as to help demonstrate how it could be used in other regions.

## 3.3	Summing Up the Methodology

It is important to note that at appropriate points throughout the following sections, the weaknesses and strengths of any other given methodology or collected data have been discussed. Only the key economic model methodologies have been discussed in this section.

Ultimately the methodology combination described above in Figures 11 and 12, enables all objectives to be met, whilst achieving the dissertation aim.

# STAGE A

# Previous Feasibility Studies

This chapter explores previous feasibility studies and assesses their methodologies, so that useful approaches can be adopted for the feasibility model within this dissertation.

## 4.0	Past Feasibility Studies Related to Algae Biodiesel Production

Previous studies have assessed biodiesel production cost (You et al, 2008); and profitability (Gallagher, 2010); for many types of feedstock including algae (Behzadi and Farid, 2007; Ugwu et al, 2008); with hybrid power plant set-up (Dimitrov, 2007); under differing ownership schemes and scales (Whittington, 2006; Grant, 2003); and, also across a variety of geographic locations worldwide (Lee and Han, 2008). However, there are very few that focus specifically on algae biodiesel plants. This section now reviews the methodologies, and summarises key findings of these selected economic feasibility assessments.

## 4.1	Methodologies for the Analysis of Economic Feasibility

An economic feasibility analysis is defined as a comparison of anticipated costs and revenues associated with a planned business venture (Dobbs, 1988). It requires four components: 1-cost estimation; 2-revenue estimation; 3-analysis of potential markets, demand, and competition; and 4-expected profit or loss, and break-even points (Dobbs, 1988). On a microeconomic level, the output is an enterprise budget (estimate of costs and revenues) for a business proposition (Norman et al, 1995). The study should evaluate the risks arising from the inherent uncertainty of assumptions, as well as unpredictable variables. An extension of the economic feasibility analysis involves sensitivity analysis, where partial budgets are used in order to estimate profit sensitivity as costs or revenues vary (Norman et al, 1995). The scope of previous biodiesel production studies indicates five key impacts on profitability:

1. feedstock cost

2. biodiesel market access and potential

3. the value of waste or co-products

4. governmental regulation

5. utility costs

All selected studies use spreadsheet budget-simulations, and tend to find that biodiesel production is not economically feasible at present, in comparison to fossil diesel.

## 4.2	Derived Major Economic Costs and Benefits of Production

Drawing inferences from the selected studies, as well as data from oilgae.com, significant cost and revenue components of production are shown below, in table 1.

Table 1: Cost and revenue components of algae biodiesel production

| Costs |  | Valuable Products and Extra Revenues |
| --- | --- | --- |
| Fixed | Variable |  |
| Plant Building | Maintenance | Biodiesel |
| Algae PBR | CO2 | Algae Waste |
| Biodiesel Reactor | Nutrients (fertilisers) | Glycerine |
| Biodiesel Storage | Methanol | Government Funding |
| Algae Oil Storage | Catalyst (KOH) |  |
| Algal Seeding Medium Storage | Electricity |  |
| Algae Waste Storage | Labour |  |
| Electricals and Pumps | Testing Fees and Supplies |  |
| Insurance |  |  |
| Legal Advice and IP |  |  |

# STAGE B

# Review of Photobioreactor Production Systems

This chapter gives an overview of algae cultivation methods and their technical feasibilities.

## 5.0	Introduction

This section analyses the most efficient production configuration for microalgal biomass, forms the underlying foundation upon which the future contribution of algae technology relies. The technical systems for the production of algae are known as photobioreactors. The two distinct families of photobioreactor systems will be analysed and compared in this section, to form  productive and economic viability understanding, upon which to base decisions for development. The most obvious difference in microalgal production systems is whether the microalgal culture is exposed to the environment. Open systems consist of simple, widespread raceway ponds, whilst closed systems consist of tubular shapes of varying size, shape, orientation, and transparent materials used (Tredici, 1999). The key success factors affecting solutions for optimum growth are light exposure and turbulence (Tredici, 1999).

## 5.1	Technical Comparison of Algae Production Systems

Table 2: Example of Technical Options of Cultivation Equipment

A number of technical solutions distinguish between open systems, and closed systems (see table 2 below).

| Open Production System |  | Closed Production System |  |
| --- | --- | --- | --- |
| Physical | Technical | Physical | Technical |
| Cuvette, container, stirred vessel | Material (glass, plastics) | Plastic sleeves | O2 removal > CO2 input |
| Natural water | Turbulence development (pumping, stirring) | Fermenter-like tank | Type and duration of illumination |
| Raceway pond | Flow direction (horizontal/vertical) | Tubular PBR | Temperature control |
| Inclined surface device | Surface to volume-ratio | Laminar PBR | Sterilisation |

Source: Pulz, 2001

## 5.2	Open Systems

Common open pond systems are raceway ponds driven by paddle wheels, and sometimes waterjets (Tredici, 1999). They usually operate at water depths of 15–20 cm (see figure13 and 14 below) (Tredici, 1999). The baffles on the diagram function to increase the mixing of the algae medium (Tredici, 1999).

Figure 14: Example of Real Open Raceway Pond

Figure 13: Birds Eye Schematic of Open Raceway Pond

Source: Steger, 2009

Source: Chisti, 2007

The key disadvantage is that light penetration becomes an increasingly limiting factor to growth as the pond surface algal layer increases in thickness (Pulz, 2001). This places a considerable constraint on productivity levels. Technically it is possible to enhance light intensity to the algae culture by reducing the layer thickness to a few centimeters (Pulz, 2001). This can be achieved via thin layer inclined culture systems (Pulz, 2001). Other problems with open systems include significant waste through evaporation, e.g. the diffusion of valuable CO2 or water to the atmosphere (Pulz, 2001). There is also the permanent threat of contamination and pollution that makes it difficult to maintain dominance of the desired microalgae species (Edwards, 2008). The requirement for large ground area is also an issue (Edwards, 2008).

### 5.2.1	Closed Ponds

A variation on the open pond design is known as closed pond. This is a quasi-closed system that grants better environmental control. Closed pond systems are considerably cheaper per area than closed photobioreactor designs, and only cost slightly more than open ponds (Kizililsoley and Helvacioglu, 2008).

The set-up is similar to open pond designs but simply covered with an added greenhouse structure (usually made of plexiglass panels and metal frames) (Kizililsoley and Helvacioglu, 2008). It allows more species to be grown as compared to pure open pond systems, as well as greater assurance of desirable species, whilst extending the growing season dramatically when heated (it usually enables year long production) (Kizililsoley and Helvacioglu, 2008). It is also possible to increase the amount of CO2 in closed ponds, which promotes greater algae yield (Kizililsoley and Helvacioglu, 2008). However, it is not 100% closed and therefore does not guarantee any of these functions.

## 5.3	Closed Systems

Closed systems allow complete control of practically all the important biotechnical growth variables (Pulz, 2001). It is difficult to maintain uniform conditions for algae growth in open systems, which is a key area where closed systems are more desirable (Pulz, 2001). They allow reproducible production conditions which meet quality good manufacturing practice, and therefore make ISO accreditation possible (Pulz, 2001).

The closed system has evolved into four general form factors as explained below.

### 5.3.1	Flat-Plate Photobioreactors

These create the largest surface area for light absorption (Pulz, 2001). They can be set-up with natural or artificial, fluorescent light sources, and have branched into Vertical alveolar panel (VAP) which consists of many 1.6-cm-thick Plexiglas alveolar sheets, and flat-plate design (Tredici and Materasi, 1992). They tend to be made of transparent materials such as PVC, for maximum transmittance of solar light energy to the algal medium (Tredici and Materasi, 1992). Flat panel designs allow high photosynthetic efficiency with low oxygen accumulation compared to other photobioreactors (Takahama U, 1982). This reduces the risk of photobleaching of the algae chloroplasts at high dissolved oxygen concentrations, which would otherwise disrupt the algae’s ability to turn light energy into useful biomass (Takahama U, 1982). See figure 15 and 16 below for an idea of the set-up.

Figure 16: Schematic for an Inclined Horizontal Alveolar Panel Photobioreactor

Figure 15: Schematic for a Vertical Alveolar Panel Photobioreactor

Source: Tredici and Materassi, 1992

Source: Tredici and Materassi, 1992

### 5.3.2	Tubular Photobioreactors

Tubular photobioreactos consist of arrays of straight transparent, plastic tubes (can sometimes be glass) (Pulz, 2001). A tubular array is called a solar collector, and is where sunlight is captured by the algae (Pulz, 2001). The solar collector tubes are limited to a diameter of ≤10cm because of poor light penetration through the dense algae medium easily obtained with this design (Chisti, 2007). This medium circulates between a reservoir and the solar collector, which allows continuous operation (Chisti, 2007). Circulation and mixing are driven preferentially by airlift system, or alternatively by a pump (however this tends to damage the biomass) (Chisti, 2007). Orientations of the array can assume vertical, horizontal, inclined, or even bent and coiled formations when flexible plastics are used (Ugwu et al, 2008).

One key limitation that does exist with this photobioreactor design is poor mass transfer (Chisti, 2007). The symptom is seen as asymmetric distribution of CO2 and oxygen concentrations along the algae flow route which leads to increasing oxygen accumulation towards the end of the circuit (Molina et al, 2001). This becomes a massive problem when scaling up to commercially viable sizes as discussed previously, due to inhibition of specific proteins in the algae’s photosynthetic apparatus (Molina et al, 2001). This is because the culture pH rises as oxygen accumulates, and consequently causes re-carbonation of the culture medium (Molina et al, 2001). This greatly increases production costs (Ugwu et al., 2008).

When a tubular photobioreactor is scaled up via widening tube diameter, illumination surface to volume ratio decreases so much, that algae at the bottom do not experience sufficient light intensities, and so their growth is limited via the light shading effect. This can be overcome with a good mixing system (Ugwu et al, 2005). Conversely, when the length of the tube is increased this has negative effects due to oxygen accumulation (Ugwu et al, 2005). The only feasible option therefore for scale-up would be to modularise the tubular arrays so that many can be rolled out adjacent to each other without breaking the circuit length and diameter constraints that offer highest productivity.

Another issue is temperature control. Thermostats can be used to maintain optimal culture temperature, but they can be expensive and difficult to implement (Ugwu et al., 2008). A final problem arises from adherence of the algae cells to the inner surface of the tubes (Chisti, 2007). Such biomass sedimentation can be minimised through maintenance of a highly turbulent flow (Chisti, 2007).

A typical example of this type of system can be seen in figure 17 below.

Figure 17: Schematic of Horizontal and Vertical Arrays of Tubular Photobioreactors

Source: Hosikian et al, 2010

### 5.3.3	Vertical-Column Photobioreactors

Vertical-column designs are cheap, compact, and simple to run (Miron et al., 2002). They seem promising for large-scale algae production, and yet can attain algae concentrations and specific growth rates that are typically common for narrower photobioreactors (Miron et al., 2002). A dominant design called the bubble column has draft tubes, or is alternatively constructed as a split cylinder, in order to allow the mixing cycle to form within the reactor vessel (Ugwu et al., 2008). The schematic for this design can be seen on Figure 18 below.

Figure 18: Vertical Column Photobioreactor

Source: Choi et al, 2003

#### 5.3.3.1	Internally-illuminated photobioreactors

It may be desirable to internally illuminate certain photobioreactor designs with fluorescent lamps. Figure 19 below shows this set-up can be achieved. The impellers mix the algal medium, and gases are supplied via a sparger through the bottom of the photobioreactor (Ugwu et al., 2008). The photobioreactor can in effect be illuminated 24 hours a day, via solar light during the daytime and artificial light during the night time (or even on cloudy days). One downside however, is that mass cultivation of algae using sets of this photobioreactor design would require considerable technical expertise (Ugwu et al., 2008).

It is possible to install optic fiber apparatus that can transmit light from the external illumination surface through to the middle of wider vessels in order to overcome the light shading effect (Ugwu et al., 2008). This technique can have a considerable effect on the productivity of a photobioreactor.

Figure 19: Internally Illuminated Photobioreactor

Source: Ugwu et al., 2008

### 5.3.4	Oscillatory Flow Photobioreactor

This is the most cutting edge design when it comes to closed photobioreactors, and comes in two forms (Reis, 2006). The batch micro-bioreactor, and the continuous meso-reactor (Reis, 2006). An example of the meso-reactor setup can be seen below in figures 20 and 21. For this design, an array of SPC tubes of around 800 mm length, are near-horizontally positioned and connected in series. Fluid is oscillated using a piston movement produced via an electromagnetic oscillator. The piston is attached to a membrane of the ‘mixing chamber’ that adjoins the base of the array (Reis, 2006). The advantageous thing about the set-up is that the mechanism of mixing is very low energy and causes little hydrodynamic stress on algae being grown (Reis, 2006). This ultimately results in very high productivities and lower operational costs. Preliminary figures suggest an 86% improvement in yield with this design, in comparison to other closed photobioreactors (Reis, 2006).

Figure 21: Simplified Schematic of the Oscillatory Flow Photobioreactor

Figure 20: Oscillatory Flow Photobioreactor

Source: Reis, 2008

Source: Reis, 2006

## 5.4	So Where Should the Focus Transition Towards, and Why is it Focusing onto Closed?

All types and variations on types of photobioreactor have been brought to pilot scale at some point in the past, but the latest developments show a trend focusing onto those of the tubular and flat-plate design, alongside an array of lighting adjustments to eradicate the light shading effect (improving uniform light intensity of the photobioreactor) (Chisti, 2007). This is tending to be done more via lamination of the photobioreactors along a north to south orientation (to funnel as much sunlight inwards as possible) (Chisti, 2007). This should mean better productivity under conditions where light is of lower ambient intensity, and it means that lower circulation or mixing rates are required and so avoids unnecessary algae damage from turbulence (Ugwu et al., 2008). This lamination principle being pursued is modeled on how higher plants in general improve their total light receiving surface area by having many leaves, and therefore being much more productive with the light available. Eventually, oscillatory designs should become preferable as learning and expertise increases.

## 5.5	Summary of Photobioreactor Designs

Due to the previous explanations of open and closed systems, there are a number of advantages and disadvantages that must be taken into account when designing a production system of either type. These issues are highlighted on table 3 below, in a comparison between general properties of open and closed systems. It allows you to determine the suitability of a design based on your chosen concept, geographic area, and algae strain. There seems to be the clear dominance of closed system preference in terms of advantages.

Table 3: Advantages and Disadvantages of Open and Closed Algal Cultivation Plants

| Parameter | Open ponds (raceway ponds) | Closed systems (PBR systems) |
| --- | --- | --- |
| Contamination risk | Extremely high | Low |
| Space required | High | Low |
| Water losses | Extremely high | Almost none |
| CO2-losses | High | Almost none |
| Biomass quality | Not susceptible | Susceptible |
| Variability as to cultivatable species | Not given, cultivation possibilities are restricted to a few algal varieties | High, nearly all microalgal varieties may be cultivated |
| Flexibility of production | Change of production between the possible varieties nearly impossible | Change of production without any problems |
| Reproducibility of production parameters | Not given, dependent on exterior conditions | Possible within certain tolerances |
| Process control | Not given | Given |
| Standardisation | Not possible | Possible |
| Weather dependence | Absolute, production impossible during rain | Insignificant, because closed configurations allow production also during bad weather |
| Period until net production is reached after start or interruptions | Long, approx. 6–8 weeks | Relatively short, approx. 2–4 weeks |
| Biomass concentration during production | Low, approx. 0.1–0.2 g/l | High, approx. 2–8 g/l |
| Efficiency of treatment processes | Low, time-consuming, large volume flows due to low concentrations | High, short-time, relatively small volume flows |

Source: Pulz, 2001

Moving on from this it has been possible for (Chisti, 2007) to aggregate average productivities of both key closed systems and open systems. The comparison shows annual production level of 100 tonnes of biomass for average closed and average open systems. On top of this, both production methods are assumed to have input an identical amount of carbon dioxide, because losses to the atmosphere are ignored. Figures can be seen on table 4 below. The table shows final yields that favour closed systems at this scale.

Table 4: Basic values of production for Open and Closed Cultivation Plants

| Variable | Photobioreactor facility | Raceway ponds |
| --- | --- | --- |
| Annual biomass production (kg) | 100,000 | 100,000 |
| Volumetric productivity (kg m− 3 d− 1) | 1.535 | 0.117 |
| Areal productivity (kg m− 2 d− 1) | 0.048a | 0.035b |
|  | 0.072c |  |
| Biomass concentration in broth (kg m− 3) | 4 | 0.14 |
| Dilution rate (d− 1) | 0.384 | 0.25 |
| Area needed (m2) | 5681 | 7828 |
| Oil yield (m3 ha− 1) | 136.9d | 99.4d |
|  | 58.7e | 42.6e |
| Annual CO2 consumption (kg) | 183,333 | 183,333 |
| System geometry | 132 parallel tubes/unit; 80 m long tubes; 0.06 m tube diameter | 978 m2/pond; 12 m wide, 82 m long, 0.30 m deep |
| Number of units | 6 | 8 |
| a Based on facility area. |  |  |
| b Based on actual pond area. |  |  |
| c Based on projected area of photobioreactor tubes. |  |  |
| d Based on 70% by wt oil in biomass. |  |  |
| e Based on 30% by wt oil in biomass. |  |  |

Source: Chisti, 2007

The data was originally from a 2007 study, and can therefore be updated to take into account the latest advances and achievements for each type of system (taken into account later in the economic model).

# STAGE C

# Building a Model to Ascertain the Economic Viability of Algae Biodiesel Projects

This chapter explores whether commercial scale biodiesel production is economically feasible in the UK, and compares estimates of total production costs for two size configurations of algae biodiesel plants (one at 100,000L per annum, and another at 10,000,000L per annum). The assessment will analyse the various cost pools involved in an algae biodiesel venture as outlined in the following section (6.0).

## 6.0	The Algae Biodiesel Plant Set-up

The photobioreactor technicalities have been previously covered, but it is important to show it in the context of the whole operation that is proposed. There are three main cost pools as can be seen on Figure 22 below. It is a diagram giving an overview of the general flow of inputs through to output (biodiesel) for a proposed algae biodiesel plant, as well as the intermediates along the process.

Figure 22: The Cost Pools Associated with Algae Biodiesel Production

Source: Author

With productivities and photobioreactor designs assessed, it is possible to now go on to analyse the economic viability of the available photobioreactor systems, and compare them based purely on economics. In reality it may be prudent to also take into account life cycle analysis of greenhouse gases, in order to balance competing priorities of profit and environmental protection, but for the following model only the economic factors will be addressed. Cost for production of biomass has been sufficiently studied by (Chisti 2007), so the main comparative cost pools studied in this dissertation will be the extraction of algae oil, and the conversion of algae oil into biodiesel (cost pools 2 and 3).

## 6.1	The Economic MODEL

The constructed Economic Model is refreshed below in Figure 12b.

Figure 12b: Schematic of the Modules Comprising the Economic Viability Model

Source: Author

## 6.2	Data Collection for the Economic Model

Data has been collected to supply inputs for the modules of the economic model outlined above. The framework aims to estimate costs, revenues, and expected profit or loss. Separate budgets for crop production, oil extraction, and biodiesel production are calculated. Crop yields have been derived as shown previously, whilst all variable and fixed costs have been obtained directly from online data and literature, and referenced in the text. A list of calculation conversions can be seen in APPENDIX C.

The enterprise budget methodology that (Chisti, 2007) uses to assess biomass production costs (i.e. cost pool 1) follows that of (Molina Grima et al., 2003). The components of costs taken into account included the charges related to capital investment and all operating expenses.

# Total Cost For Algae Biodiesel Plants

## 7.0	Introduction

This provides inputs for the NPV module that are assumed to be fixed during the entire project life-cycle. This is obviously not the case in reality because we know that all values change as time moves forward. Despite this, it is useful to approach with this method in order to reduce complexity. The study will assess the prospects of two different sized pants. One with an output of 0.1 million litres biodiesel per annum, and another with an output of 10 million litres per annum. According to the Department of Energy and Climate Change projections, the 10 million litre configuration plant should be capable of supplying roughly 0.5% of the average annual UK biodiesel demand, projected over the next 10 years (UK Department of Energy and Climate Change, 2009).

The idea is that the algae cultivation capital will be rolled out in a modular fashion in increments of the standard 10,000 tonne biomass plant that (Chisti, 2007) estimated price at, so that the same economies of scale and properties or ratios of the plants are used. This should enable retention of the credibility and usability of the results.

The method of choice for estimation of cost pool 2 is enterprise budget analysis based on flowsheets, layouts, and preliminary equipment descriptions and specifications. According to (Humphreys, 2004), this technique has an accuracy range of +30% to -15%. Cost pool 3 is estimated via a scale function (Crockett, 2006).

CO2 gas variable costs are estimated as zero because it works on the provision of free CO2 sourced from locations near heavy CO2 factory or power plants. As the algae biodiesel venture would be removing many tones of CO2 from their annual emissions, it will save them money and therefore the only cost involved for the venture should arise from the initial fixed machinery installation required to harvest and purify the CO2 from exhaust fumes. Eventually as carbon emissions permit prices increase on the Energy Trading Scheme (ETS) (EFMA, 2007), it may become possible to even treat CO2 input as a variable revenue.

Economies of scale coefficients have been introduced to the budget statement for cost pool 2, because studies have shown that they have a significant role in capital intensive production industries such as that of algae biodiesel. A key study was undertaken by Haldi and Whitcomb that classified the typical scale coefficients that have been used. An exponential function has been applied to both major equipment at 0.89, and labour requirements at 0.79 (Haldi and Whitcomb, 1967). This is to reflect the reality of economies of scale associated with certain components of algae biodiesel plants. The function is in the form:

C=aXb

Where:

C represents cost,

X represents output capacity

a is a constant

b is an exponent known as the scale coefficient.

If b < 1 this indicates increasing returns to scale. If b = 1 this indicates constant returns to scale. If b > 1 this indicates decreasing returns to scale (Haldi and Whitcomb, 1967). Values for the scale coefficients used are from tables 1 and 3 (Haldi and Whitcomb, 1967). Labour especially enjoys scale returns through specialisation efficiency gains.

## 7.1	Adjusting for Oil Content

Table 5: Oil Content Percentages of a Selection of Major Algae Species

Microalgae with high oil content percentages are desirable for biodiesel production. A list of commonly assessed species and their corresponding oil productivities are given on table 5 below.

| Microalgae Species | Oil Content /% Dry Weight |
| --- | --- |
| Crypthecodinium cohnii | 20 |
| Monallanthus salina | > 20 |
| Tetraselmis sueica | 15–23 |
| Dunaliella primolecta | 23 |
| Phaeodactylum tricornutum | 20–30 |
| Chlorella sp. | 28–32 |
| Isochrysis sp. | 25–33 |
| Nannochloris sp. | 20–35 |
| Cylindrotheca sp. | 16–37 |
| Nitzschia sp. | 45–47 |
| Neochloris oleoabundans | 35–54 |
| Nannochloropsis sp. | 31–68 |
| Botryococcus braunii | 25–75 |
| Schizochytrium sp. | 50–77 |

Source: Chisti, 2007

So the 70% algae biomass oil yields are assumed because the aim is to explore the best possible option.

The budget does not take into account the initial, one-time cost of acquiring the algae strain seeding medium. The analysis also assumes that recycling of alcohol and acetyl chloride takes place.

## 7.2	Total Production Cost

### 7.2.1	Algae Biomass Production Costs (Cost Pool 1)

The assumed cost per kilogram of biomass can be seen in Table 6, below. The estimation methods followed by (Chisti, 2007) can be viewed in (Humphreys, 2004 and Molina Grima et al., 2003). The values include all associated fixed and variable costs.

Table 6: Algae Biomass Production Costs

| Configuration /L biodiesel output | Initial capital investment portion | Annual production cost |
| --- | --- | --- |
| 100,000 | 144104.84 | 40270.16 |
| 10,000,000 | 2295907.62 | 641592.38 |

### 7.2.2	Algae Oil Extraction Costs (Cost Pool 2)

Here the methodology and cost data was derived from (Molina Grima et al., 2003). The following tables give a walkthrough of how the extraction costs were calculated from major economic costs of equipment.

| Table7: Lists of Major Equipment Costs for Crude Esterified Algae Oil extraction |  |  |  |  |
| --- | --- | --- | --- | --- |
| 100,000L configuration |  |  |  |  |
| Item | Delivered cost /£ | No. of units | Total Cost | % of MEC |
| 1. Ethanol tank (1.5 m3, carbon steel) | 1503 | 3 | 4232 | 0.05 |
| 2. Acetyl chloride tank (1 m3, carbon steel) | 1176 | 3 | 3311 | 0.04 |
| 3. Reactor (2 m3, 66 psig, jacketed) | 3276 | 3 | 9229 | 0.10 |
| 4. Hexane storage tanks (60 m3) | 7258 | 6 | 40889 | 0.44 |
| 5. Holding tank (2.5 m3, carbon steel) | 2146 | 3 | 6046 | 0.07 |
| 6. Evaporator (50 ft2) | 9139 | 3 | 25742 | 0.28 |
| 7. Condenser (shell and tube, 5 m2) | 1049 | 3 | 2954 | 0.03 |
| Total MEC /£ |  |  | 92403 |  |
| 10,000,000L Total MEC /£ |  |  | 2627007 | * |
| *using scale coefficient from (Haldi and Whitcomb, 1967) |  |  |  |  |

| Table 8: Lists of Total and Annual Fixed Capital Costs for Algae Oil Extraction |  |  |
| --- | --- | --- |
| 100,000L configuration |  |  |
| Item | Cost /£ | % of A |
| 1. Major purchased equipment (MEC) | 92,403 | 0.383 |
| 2. Installation costs (at 0.3 MEC) | 27,721 | 0.115 |
| 3. Instrumentation and control (at 0.1 MEC) | 9,240 | 0.038 |
| 4. Electrical and Piping (at 0.3 MEC) | 27,721 | 0.115 |
| 6. Buildings (at 0.3 MEC) | 27,721 | 0.115 |
| 7. Yard improvements (at 0.1 MEC) | 9,240 | 0.038 |
| 8. Service facilities (at 0.2 MEC) | 18,481 | 0.077 |
| 9. Land (at 0.06 MEC) | 5,544 | 0.023 |
| 10. Engineering and supervision (at 0.25 MEC) | 23,101 | 0.096 |
| Total fixed capital investment, A /£ | 241,172 |  |
| Item | Cost /£ | % of B |
| Depreciation (at (sum of items 1–8)/12 years) | 17,711 | 0.984 |
| Property tax (at 0.01 depreciation) | 177 | 0.010 |
| Insurance (at 0.006 depreciation) | 106 | 0.006 |
| Total fixed capital per year, B /£ | 17,994 |  |
| 10,000,000L configuration |  |  |
| Item | Cost /£ | % of A |
| 1. Major purchased equipment (MEC) | 2,627,007 | 0.383 |
| 2. Installation costs (at 0.3 MEC) | 788,102 | 0.115 |
| 3. Instrumentation and control (at 0.1 MEC) | 262,701 | 0.038 |
| 4. Electrical and Piping (at 0.3 MEC) | 788,102 | 0.115 |
| 6. Buildings (at 0.3 MEC) | 788,102 | 0.115 |
| 7. Yard improvements (at 0.1 MEC) | 262,701 | 0.038 |
| 8. Service facilities (at 0.2 MEC) | 525,401 | 0.077 |
| 9. Land (at 0.06 MEC) | 157,620 | 0.023 |
| 10. Engineering and supervision (at 0.25 MEC) | 656,752 | 0.096 |
| Total fixed capital investment, A /£ | 6,856,489 |  |
| Item | Cost /£ | % of B |
| Depreciation (at (sum of items 1–8)/12 years) | 503,510 | 0.984 |
| Property tax (at 0.01 depreciation) | 5,035 | 0.010 |
| Insurance (at 0.006 depreciation) | 3,021 | 0.006 |
| Total fixed capital per year, B /£ | 511,566 |  |

| Table 9: Lists of Direct Costs of Oil Extraction |  |  |  |
| --- | --- | --- | --- |
| 100,000L configuration |  |  |  |
| Raw materials | Total quantity | Cost /£ | % of C |
| 3. Hexane, m3 (at £227.94/m3) | 5.6 | 1284.11 | 1 |
| Total raw materials cost, C /£ |  | 1,284 |  |
| Utilities | Total quantity | Cost /£ | % of D |
| 4. Cooling water, m3 (£0.018375/m3) | 16,309 | 299.68 | 0.432 |
| 5. Steam, kg (at £0.003063/kg) | 110,700 | 339.01875 | 0.489 |
| 6. Power, kW h (at £0.036769/kW h) | 1,476 | 54.270675 | 0.078 |
| Total, D /£ |  | 692.97 |  |
| Others Total quantity | Total quantity | Cost /£ | % of E |
| 7. Labor (at £10/h, 1 shift) | 2.8 | 58,589 | 0.439 |
| 8. Supervision (at 0.2 labor) | employees | 11,718 | 0.088 |
| 9. Payroll charges (at 0.25 (labor + supervision)) |  | 17,577 | 0.132 |
| 10. Maintenance (at 0.04 MEC) |  | 3,696 | 0.028 |
| 11. Operating supplies (at 0.004 C) |  | 5 | 0.000 |
| 12. General plant overheads (at 0.55 |  | 40,702 | 0.305 |
| (labor + supervision + maintenance)) |  |  |  |
| 13. Tax (at 0.16 items 1–6, 10 and 11) |  | 1,114 | 0.008 |
| 14. Contingency (at 0.05 items 1–7) |  | 163 | 0.001 |
| 15. Marketing (not included) |  | 0 | 0 |
| Total, E /£ |  | 133,564 |  |
| Total production cost, F (B (Table 4) +C +D+ E) /£ |  | 153,535 |  |
| Unit cost of extracting oil (£/L) |  | 1.54 |  |
| 10,000,000L configuration |  |  |  |
| Raw materials | Total quantity | Cost /£ | % of C |
| 4. Hexane, m3 (at £227.94/m3) | 761.4 | 173542.22 | 1 |
| Total raw materials cost, C /£ |  | 173,542 |  |
| Utilities | Total quantity | Cost /£ | % of D |
| 5. Cooling water, m3 (£0.018375/m3) | 2,204,134 | 40500.96 | 0.432 |
| 6. Steam, kg (at £0.003063/kg) | 14,960,700 | 45817.14 | 0.489 |
| 7. Power, kW h (at £0.036769/kW h) | 199,476 | 7334.48 | 0.078 |
| Total, D /£ |  | 93652.58 |  |
| Others Total quantity | Total quantity | Cost /£ | % of E |
| 8. Labor (at £10/h, 1 shift) | 226.6 | 4,713,786 | 0.451 |
| 9. Supervision (at 0.2 labor) | employees | 942,757 | 0.090 |
| 10. Payroll charges (at 0.25 (labor + supervision)) |  | 1,414,136 | 0.135 |
| 11. Maintenance (at 0.04 MEC) |  | 105,080 | 0.010 |
| 12. Operating supplies (at 0.004 C) |  | 694 | 0.000 |
| 13. General plant overheads (at 0.55 |  | 3,168,893 | 0.303 |
| (labor + supervision + maintenance)) |  |  |  |
| 14. Tax (at 0.16 items 1–6, 10 and 11) |  | 87,442 | 0.008 |
| 15. Contingency (at 0.05 items 1–7) |  | 22,037 | 0.002 |
| 16. Marketing (not included) |  | 0 | 0 |
| Total, E /£ |  | 10,454,825 |  |
| Total production cost, F (B (Table 4) +C +D+ E) /£ |  | 10,722,019 |  |
| Unit cost of extracting oil (£/kg) |  | 1.07 |  |

### 7.2.3	Transesterification Costs (Cost Pool 3)

In the study by (Chisti, 2007) the estimation of biodiesel transesterification costs was insufficient, as no detailed analysis of data took place. An assumption was made which is likely to be very wrong because they just used the gap between the market price for biodiesel and the cost for production of the feedstock oil. In reality the market price is dictated by the aggregate influences of all possible feedstocks (as well as other external factors) and so using such an arbitrary figure on algae biodiesel production ventures has been considered inaccurate. For this reason it will be more specifically calculated below. It takes into account the necessary fixed capital cost, as well as annual variable and fixed costs of the biodiesel production unit.

Estimated capital costs of contracted biodiesel reactor machinery have been derived from (Crockett, 2006), and exclude costs arising from land, buildings, utility installation, wastewater treatment, and rail track development which will be covered separately. The data has also been adjusted for a 5% annual technology learning rate as stipulated by the International Energy Agency (IEA, 2009). This has occurred via a compound percentage cost decrease of 5% over four years because the study was completed in 2006, in order to estimate current feasible production efficiencies.

Table 10: Current Feasible Biodiesel Production Cost Efficiencies Depending on Capacity

| Annual Oil Production /million L | Capital cost $/L | Capital cost £/L |
| --- | --- | --- |
| 2.27 | 0.36 | 0.22 |
| 4.55 | 0.25 | 0.16 |
| 22.73 | 0.16 | 0.10 |
| 90.92 | 0.13 | 0.08 |

These calculations form the scale factor curve for capital cost £/L as can be seen in Figure 23, below.

Figure 23: Scale Factor Curve for Capital Cost

Capacity /Litres

Cost /£

Plotting a power trendline across the data allows you to estimate the costs for both plant sizes in this economic analysis by plugging production figures into the function’s equation. The result is seen below, in table 11. The 70% biomass oil content has been taken into account in the cost.

Table 11: Initial Costs of Biodiesel Reactor Capital

| plant size / kg biomass production | Initial Capital cost for Transesterification Reactor /£ |
| --- | --- |
| 100000 | 986.2798 |
| 10000000 | 27289.7869 |

Variable cost assumptions can now be derived from data in table 15 of (Crockett, 2006). Methanol price is derived from data on the Methanex trade price tracker which suggests a cost of 225 Euros per MT in the EU over the period of 1st July until 30th September (Methanex, 2010). This was set on the 6th July, and at this time 1 EUR was worth around 0.83 GBP (X-rates, 2010). Catalyst price is derived from the average of data on (ICIS, 2010). The figure used is a 12 month average of 628 $/short ton. Methanol and catalyst freight is calculated based on a proviso of $4,200 per 30,000 gal rail car (Crockett, 2006), with the assumption that any plant site is localised near a rail depot. It is important to note that it is assumed a mistake has been made on table 15 of this study, for the catalyst freight cost, which for the purpose of this model is corrected to 0.10 $/kg. Operational costs are inferred from (Gerpen et.al., 2005). The calculated data is shown on Table 12, below.

Table 12: Variable Costs for Biodiesel Transesterification Process

| Item | Unit | Cost | Assumed Cost | Units of assumed cost |
| --- | --- | --- | --- | --- |
| Methanol | €/MT | 225.00 | 0.19 | £/L |
| Catalyst | $/short ton | 628.00 | 0.04 | £/kg |
| Methanol Freight | $/gallon | 0.14 | 0.02 | £/L |
| Catalyst Freight | $/kg | 0.10 | 0.01 | £/kg |
| Operational Costs | $/gallon | 0.25 | 0.03 | £/L |

To determine assumed transesterification variable cost per litre of algae oil, the reaction chemical ratio data from Figure 5 in section 2.4.3 of this dissertation, has been used in combination with calculated values from Table 12, above. The result is shown below in table 13.

Table 13: Variable Cost per Litre of Biodiesel from Cost Pool 3

| Item | Assumed Transesterification Cost /L algae oil |
| --- | --- |
| Methanol | 0.02241 |
| Catalyst | 0.00040 |
| Methanol Freight | 0.00231 |
| Catalyst Freight | 0.00014 |
| Operational Costs | 0.03437 |
| Total Transesterification Cost /L Biodiesel | 0.05962 |

Following on from this it is possible to derive total variable costs for each biodiesel plant configuration under study. This is shown on table 14, below. The loss of volume of biodiesel due to the reaction is only 1%, so for the purpose of this model it is treated as no loss.

Table 14: Total Variable Cost per Litre of Biodiesel from Cost Pool 3

| plant size / kg biomass production | Total Variable Costs |
| --- | --- |
| 100000 | 4173.646039 |
| 10000000 | 417364.6039 |

## 7.3	Byproduct Revenues Function

As has already been stated, the only valuable outputs considered in this model apart from biodiesel, are algae waste biomass, and glycerin. Government biofuel duty incentives have recently been withdrawn (Massey, 2009). An aggregate revenue function is shown on table 15, below. They are calculated from data derived in (Crockett, 2006). These values are not affected by the size of production plant so apply to all configurations.

Table 15: Aggregate Revenue Function for Sale of By-Products

| Item | value /$ per kg | kg byproduct /L biodiesel | Value of byproduct per litre biodiesel produced /£ |
| --- | --- | --- | --- |
| Algae Biomass | 0.12 | 0.01 | 0.0009 |
| Glycerin | 0.46 | 0.10 | 0.0303 |
|  |  | Total | 0.0312 |

### 7.3.1	Total Annual Production Cost for Cost Pool 3

So by combining the total byproduct revenue per litre of biodiesel produced, with the costs of producing each litre of biofuel you can derive a more credible total variable cost function for each of the biodiesel plant configurations. This can be seen on Table 16, below. NB the annual costs associated with just the production of biodiesel, and not from selling biodiesel.

Table 16: Total Annual Production Cost for the Transesterification of Algae Oil into Biodiesel

| plant size / kg biomass production | Total Biodiesel Production Costs per annum |
| --- | --- |
| 100000 | 1058.285025 |
| 10000000 | 105828.5025 |

## 8.0	Project Assumptions

Project assumptions will now be outlined so that the profitability of the plant configurations can be assessed. General assumptions have been made based on the technology in use, and this covers project lifetime, construction period, and the nature of investment capital. Decommissioning percentage, and the scrap value percentage are assumed equal in that the inherent value of the plant capital at the end of the project can cover the costs of decommissioning, and so can be omitted from the calculation. Table 17, below, summarises the data.

Table 17: Project Assumptions

| plant life time | 12 |
| --- | --- |
| construction time | 1 |
| Investment capital | 100% venture |

Biodiesel plant life-span was derived from (Molina, 2001). Construction time is assumed as 1 year according to (Anderson et al., 2003).

# Diesel Price Scenarios

## 9.0	Diesel Forecast

It is possible to derive a point in time where algae biodiesel production will become competitive with fossil diesel. This requires the knowledge of the likely rate of change in oil price (from which diesel generally follows), and is likely to be vital to business planning for any venture. It will also give a more accurate net present value if the future most likely revenues are accurately estimated, and allows a project to take advantage of business timing for the production plant construction, and therefore achieve optimum market entry point in time and therefore lower costs and improve the overall speed at which break-even point is reached.

Figure 24: Some of the Key Variables that Affect Crude Oil Prices

In order to build the long term forecast for diesel prices, a set of historical data for average monthly retail prices since 2000 has been used (AA, 2010). From this it is possible to extrapolate out the likely trend in oil prices. However, this technique is prone to error because of the multitude of contextual variables and unforeseeable circumstances that could have a potential impact on the price of crude oil from which diesel is derived (see figure24 below).

Source: Author

Oil is a non-renewable resource, and such resources generally tend to follow an upwards price trend. Despite this, some studies have found that it may not simply follow a long term increasing trend. Rather, that it is predominantly governed by the interaction between demand and supply at any one point in time. In support of this, it has been suggested that non-renewable resource prices exhibit a stochastic trend rather than a certain increasing trend (Fattouh, 2007).

To overcome these short-comings, the quantitative time series forecast based purely on historic data, will be compared with qualitative forecasting opinions from respected international bodies such as the International Energy Agency. In this way expert judgement and predictive models can offset each others’ weaknesses.

## 9.1	Time Series Analysis

The raw data can be viewed in APPENDIX D. The linear regression time series analysis has been performed in a number of steps, and is useful because it corrects for cyclical diesel price behaviour. First the median retail price in each year was calculated from monthly average retail prices, which can be seen on Table 18, below. By using median values in the forecast it is possible for volatile price fluctuations to be mitigated in the forecast. These temporal volatile swings in price that are common in the oil market, would otherwise distort the results. Simple linear averages were then calculated per year (see Table 18, below). The forecast covers a 10 year period starting in 2010, and therefore it has been deemed appropriate to extrapolate from a data set of prices covering the previous ten years.

Table 18: Historic Median UK Diesel Retail Prices

| Year | Median Price £/L | Simple Moving Average |
| --- | --- | --- |
| 2000 | 0.823 | 0.823 |
| 2001 | 0.785 | 0.804 |
| 2002 | 0.762 | 0.790 |
| 2003 | 0.780 | 0.787 |
| 2004 | 0.828 | 0.795 |
| 2005 | 0.899 | 0.813 |
| 2006 | 0.947 | 0.832 |
| 2007 | 0.970 | 0.849 |
| 2008 | 1.176 | 0.885 |
| 2009 | 1.045 | 0.901 |

This was then plotted on a graph (see Figure 25, below).

Figure 25: Forecasted UK Diesel Retail Prices

The red line traces the simple moving average of oil price for the past ten years. Here, the simple moving average is derived by including all data preceding the year in question. For example, if the simple moving average for 2005 is being calculated, then data from 2000 through to 2005 is used. Likewise, if 2009 is the year in question, then data from 2000 through to 2009 is used. The data set has been extrapolated into a trendline that therefore represents the support level (Brock et al., 1992) below which it is highly unlikely that the oil price will ever drop, moving into the future.

The blue line traces the actual oil price for the past ten years. The data set has been extrapolated into a trendline that indicates the high price level, moving into the future. It is assumed that diesel price will not stray above this “roof” price for a prolonged period of time over the next ten years, because of the competing transport energy sources that are beginning to phase in and therefore increasing aggregate energy supply for ground transport.

The idea is that oil price will largely remain within the gap between the two lines. The equations of each trendline were used to produce the following table (Table 19) of forecasted expected diesel price boundaries that are used later as an input (mean form) for the NPV analysis.

Table 19: Forecasted UK Diesel Retail Prices

| Year | Linear Trend Median Price £/L | Linear Trend Simple Moving Average |
| --- | --- | --- |
| 2010 | 1.040 | 0.958 |
| 2011 | 1.078 | 0.969 |
| 2012 | 1.117 | 0.979 |
| 2013 | 1.155 | 0.990 |
| 2014 | 1.194 | 1.000 |
| 2015 | 1.232 | 1.011 |
| 2016 | 1.270 | 1.021 |
| 2017 | 1.309 | 1.032 |
| 2018 | 1.347 | 1.042 |
| 2019 | 1.386 | 1.053 |
| 2020 | 1.424 | 1.063 |

Obviously the time-lag between time of forecast and event is inherent in any prediction, and does mean that the prices cannot be assumed as 100% correct. However, the result from this forecast can be deemed reasonably reliable because it roughly correlates with the opinions of the International Energy Association. The IEA WEO reference opinion states that oil prices are expected to rebound with the economic recovery, reaching  more than $100 per barrel (in 2008 USD) by 2020 (IEA, 2009). This lies within the inter-quartile range of the forecast scenario above.

## 10.0	Building the Net Present Value Equation

For the purpose of this model the initial cost of capital for each plant configuration will constitute C0. C1 to Ct will be derived from the mean of the previous selling price forecast on Table 19 multiplied by the output, and then less total annual production costs (fixed and variable). The rate of return is derived from assumptions of the discount rate calculation. All input data assumes that construction of the algae biodiesel plant is a “Greenfield” venture with no alternative use for facilities (byproducts revenues already accounted for in total variable costs).

## 10.1	Compensating for Investor Risk in the Discount Rate

In this model the revenue increases each year. This is because the price of diesel is assumed equal to the price of biodiesel, and the forecast based on long run averages in Figure 25, projects increases. Since the cash flows are uncertain, they must be discounted against by factoring in a risk premium (Trevino, 2008). Assuming a normal distribution within the boundary, there is a 50% chance of the price being either side of the mean, and so the most likely price afforded by biodiesel will be the average of the boundary prices at any point in time.

### 10.1.1	How to calculate risk according to (Trevino, 2008).

rf is taken as the risk-free rate on government bonds. The risk-adjusted discount rate (ri) for project i, is given by the following equation:

ri=rf+βr market-rf

where (rmarket-rf) is the risk premium, and where β is the market risk of an asset and can be directly calculated (or sometimes alternatively derived from an asset risk class). It basically measures the responsiveness of the security’s return to movement in the market (Ross et al, 2009). It is defined as the relative volatility of an asset’s returns to those of the market, of linear function Y=a+β X. The β derived is historical, and can be used as a base estimate for the returns of the asset into the future. See the raw data in APPENDIX E (Standard and Poor’s Index Services, 2010).

The data has been plotted in a figure 26, below:

Figure 26: Diesel to Market Volatility Correlation, Used to Ascertain Beta

The β can therefore be defined as the slope of the trendline, which is 0.0341 i.e. 3.41%. However because this is almost zero, it can be argued that there is no correlation between the volatility of market return and the volatility of the price of diesel.

### 10.1.2 Calculating risk premium

Risk premium can be derived from average risk premiums in the past. Ibbotson and Sinquefield found that the average rate of return for large companies on the stock market was 12.3% over 1926-2005 (Ross et al, 2009). Over the same period, average risk free return was calculated as 3.8% (Ross et al, 2009), and so the risk premium gained is 8.5%.

Plugging all these values into the equation;
ri=rf+βr market-rf

Allows the derivation of the minimum rate of return required on a biodiesel algae plant. This shall be used as the discount rate in the NVP calculations.

See APPENDIX F for the NPV calculations and results.

# STAGE D

# Discussion and Analysis of Future Strategy

## 11.0 Decision Based on the NPV Results

For the purpose of this study, the most “feasible” venture is taken as the one which gives the greatest return per unit of investment. However to give a more balanced analysis the payback period for each configuration was assessed as well, as this is a key factor that investors would take into account. The idea is that the shorter the payback period, the more attractive the option, because the uncertainty of return on investment reduces as payback time gets shorter.

## 11.1	The 100,000L Algae Biodiesel Plant Configuration

The NPV calculated for this project over the time span of 2011 to 2023 was -£1.2m. This indicates that the project is not profitable and should therefore not currently be pursued. Pay back does not occur within the intended period of usefulness, although the NPV does increase towards positive net cashflow as the time period increases. This means that at some point after 2023 (assuming longer plant capital lifespan) the net cashflow would turn positive.

## 11.2	The 10,000,000L Algae Biodiesel Plant Configuration

The NPV calculated for this project over the time span of 2011 to 2023 was -£26.2m. This indicates that the project is highly undesirable as it makes a huge loss. Pay back does not occur within this period of usefulness.

In light of the above findings it is possible to draw key inferences. As plant size increases from the 100,000L to the 10 million litre, this causes greater net losses. For algae biodiesel technology, this indicates that the use of closed systems is not feasible, and infact under current technologies, follows a pattern of diseconomies of scale as output increases. However, it must be understood that a further option for any such project would be to undertake a refitting of the algae biodiesel plant at the end of the intended ten year lifespan, and this would be relatively cheap compared to building a new greenfield plant. Production could essentially carry on for another ten years and therefore a return on investment could be realised.  On top of this, it is possible to compare for example the long payback times associated with typical powerplant or crude oil infrastructure through to market. These can take just as long (UK Department of Energy and Climate Change, 2009).

## 11.3	Cost Pool Analysis

The level of influence that each cost component of the NPV model has on the overall project outcome is a useful understanding to obtain. This is because the greatest determinants on outcome can then be concentrated on mainly by the management of the project, as well as the employees operating the algae biodiesel plant once it has been commissioned. This also gives researchers and developers a better basis upon which to carry out targeted technology improvements. Below are figures 27 and 28, which show the breakdown of costs in each cost pool. The aggregated data from calculations forming this studies economic model can be viewed in APPENDIX G.

Figure 27: Total Cost per Litre of Biodiesel

In light of this, it is possible to see that the greatest cost per litre of biodiesel capacity arises in cost pool 2 (extraction of algae oil), followed reasonably closely by cost pool 1 (algae biomass production). Cost pool 3 (biodiesel transesterification) constitutes negligible cost to the biodiesel. Therefore more research and development effort needs to be targeted at lowering cost pools 1 and 2, and if you look at the actual breakdown of costs from tables 7, 8, and 9 earlier; management and employees in general need to focus on optimising all maintenance and operational processes. It is important however to note that cost pool 2 may have an overweighting in terms of cost in pool 2, due to the cost factor estimation method employed for annual maintenance and operational costs which are not 100% accurate. This may have skewed the cost burden towards cost pool 2 rather than cost pool 1, as previous studies suggest that cost pool 1 is likely the greatest or equal largest cost area. It is also interesting to note the lower cost per unit of biodiesel as the plant size increases, which shows economies of scale taking effect.

Below is Figure 28, showing the overall cost profiles of the two plant configurations split into their cost pool silos.

Figure 28: Total Cost Percentage of each Cost Pool

Once again it shows that the largest cost pool is 2, however this time it becomes apparent that the diseconomies of scale outweigh the economies of scale gained from increasing plant size beyond a certain point between the two configurations, because the overall cost increases. In order to confirm this further, real world studies would be required, but another option is to carry out an assessment of open systems or more likely quasi closed systems, because previous studies have indicated that they outcompete closed systems as the scale becomes very large and therefore may be able to overcome this net diseconomies of scale effect that has been estimated for the closed systems in this study.

## 12.0	Future Influences on Algae Biodiesel Success

It is important to note that the UK government has pulled all incentives for production of biofuels and has stated their intention to maintain this approach towards biofuels (as mentioned earlier). This is a key reason why the variation or contribution of incentives or indeed extra taxes on the biodiesel ventures has not been assessed. It has been deemed inconsequential for the purpose of projects within the UK. However, if occurring elsewhere, then such government regulation of biofuels would need to be taken into account as it has a big impact. For example in the USA there has been huge incentives for industrial plants producing biodiesel (Business Green, 2010). Likewise, in some countries there are negative attitudes towards biodiesel as the perception of it being more polluting than fossil fuel has permeated from the properties of first generation biodiesel, onto all types of biodiesel, even though this is likely a misguided view to hold (Edwards, 2008).

## 12.1	Application of AHP-Based Location Selection Framework

When algae biodiesel technology becomes suitable for large commercial scale implementation, location considerations must be made. The proposed optimum method by which to achieve the optimum location, whilst taking into account objective and subjective characteristics, is via an AHP Analysis. A demonstration of the AHP-based location selection process will be shown, in order to illustrate this most crucial part of the management of algae biodiesel projects.

The current tools used for location selection are too subjective to give reliability and optimisation of results. The AHP-based approach brings a higher degree of objectivity to location quantification decisions. It was developed by (Saaty, 1980), and can be adapted to locations selection in the algae biodiesel industry. The advantage is that both subjective and objective facets are incorporated, thus giving the management team a more rational basis for decisions.

The 9-point scale of relative importance that should be used to weight the factors in each pair-wise comparison can be seen on table 20, over page.

Table 20: Scale of Relative Importance for Pair-wise Comparison

| Intensity | Definition | Explanation |
| --- | --- | --- |
| 1 | Equal Importance | Both risks factors have equal effect. |
| 2 | intermediate |  |
| 3 | Moderate Importance | The risk factor has slightly more effect. |
| 4 | intermediate |  |
| 5 | Essential or Strong Importance | The risk factor has significantly more effect. |
| 6 | intermediate |  |
| 7 | Demonstrated Importance | The risk factor has considerably more effect. |
| 8 | intermediate |  |
| 9 | Extreme Importance | The risk factor is totally dominant in terms of its effect. |

Source: adapted from Dey, 2002

In this case, the subjective factors upon which quantification of location will occur are shown in the hierarchical structure in figure 29, below. This is known as the weighted factor approach, where subjective factors are assigned by preference of the project manager. They use their experience of carrying out industrial or biofuel plant projects. Relevant location factors identified from previous areas of the study such as those directly connected with the proposed projects of the economic model, and those explained previously on the Ishikawa diagram in Figure 7, are displayed in table 21, on the page after next. They will be weighted by the set of 3 subjective factors; Cost, Time, and Quality, that are seen on Figure 29, over page.

Figure 29: Hierarchical Structure

The priority of the subjective factors is now established via pair-wise comparison. They have been compared in the matrix to indicate the relative importance of the y-axis over the x-axis.

| Calculating Priority Vectors For Each Subjective Factor |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Cost | Time | Quality |  |  |
| Cost | 1.000 | 4.000 | 2.000 |  |  |
| Time | 0.250 | 1.000 | 0.500 |  |  |
| Quality | 0.500 | 2.000 | 1.000 |  |  |
| TOTALS | 1.750 | 7.000 | 3.500 |  |  |
| Divide each column by its column total... |  |  |  |  |  |
|  | Cost | Time | Quality | AVERAGES |  |
| Cost | 0.571 | 0.571 | 0.571 | 0.571 |  |
| Time | 0.143 | 0.143 | 0.143 | 0.143 |  |
| Quality | 0.286 | 0.286 | 0.286 | 0.286 |  |
| Averages indicate the priority vector for a given subjective factor with respect to the overall goal... |  |  |  |  |  |
| Cost | 0.571 |  |  |  |  |
| Time | 0.143 |  |  |  |  |
| Quality | 0.286 |  |  |  |  |

Table 21: Number Codes for the Algae Biodiesel Specific Location Factors

The priority of the algae biodiesel specific location factors is now established via pair-wise comparison. The factors in question are listed below in table 21.

| 1 Proximity to CO2 Source | 2 Proximity to Rail Line for Spur Development |
| --- | --- |
| 3 Average Regional Light Intensity | 4 Regional Prevalence of Green Attitudes |
| 5 Ease of Waste Disposal | 6 Government Support |

These location factors should now be compared via pair-wise comparison matrices, to indicate the relative importance of the y-axis over the x-axis for each of the subjective factors shown earlier on figure 29. The calculation steps to be carried out for each location factor with respect to a cost are shown below as an example, and the output is a set of location factor priority vectors for cost.

| Calculating Priority Vectors For Cost |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |  |
| 1 | 1.000 | 0.500 | 7.000 | 5.000 | 0.333 | 0.333 |  |
| 2 | 2.000 | 1.000 | 1.000 | 0.167 | 0.143 | 0.333 |  |
| 3 | 0.143 | 1.000 | 1.000 | 0.167 | 0.200 | 1.000 |  |
| 4 | 0.200 | 6.000 | 6.000 | 1.000 | 1.000 | 6.000 |  |
| 5 | 3.000 | 7.000 | 5.000 | 1.000 | 1.000 | 3.000 |  |
| 6 | 3.000 | 3.000 | 1.000 | 0.167 | 0.333 | 1.000 |  |
| TOTALS | 9.343 | 18.500 | 21.000 | 7.501 | 3.009 | 11.666 |  |
| Divide each column by its column total... |  |  |  |  |  |  |  |

|  | 1 | 2 | 3 | 4 | 5 | 6 | AVERAGES |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.107 | 0.027 | 0.333 | 0.667 | 0.111 | 0.029 | 0.212 |
| 2 | 0.214 | 0.054 | 0.048 | 0.022 | 0.048 | 0.029 | 0.069 |
| 3 | 0.015 | 0.054 | 0.048 | 0.022 | 0.066 | 0.086 | 0.049 |
| 4 | 0.021 | 0.324 | 0.286 | 0.133 | 0.332 | 0.514 | 0.269 |
| 5 | 0.321 | 0.378 | 0.238 | 0.133 | 0.332 | 0.257 | 0.277 |
| 6 | 0.321 | 0.162 | 0.048 | 0.022 | 0.111 | 0.086 | 0.125 |
| Averages indicate the priority vector for a given risk factor with respect to cost... |  |  |  |  |  |  |  |
| 1 | 0.212 |  |  |  |  |  |  |
| 2 | 0.069 |  |  |  |  |  |  |
| 3 | 0.049 |  |  |  |  |  |  |
| 4 | 0.269 |  |  |  |  |  |  |
| 5 | 0.277 |  |  |  |  |  |  |
| 6 | 0.125 |  |  |  |  |  |  |

Once location factor priority vectors have been calculated for each subjective factor, it is possible to develop an overall priority ranking for the location factors. This calculation process is illustrated below.

| Developing An Overall Priority Ranking For Each Algae Biodiesel Specific Location Factor |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Cost | Time | Quality | Overall Priority Ranking |
| 1 = | +0.571(a) | +0.143(a) | +0.286(a)= |  |
| 2 = | +0.571(b) | +0.143(b) | +0.286(b)= |  |
| 3 = | +0.571(c) | +0.143(c) | +0.286(c)= |  |
| 4 = | +0.571(d) | +0.143(d) | +0.286(d)= |  |
| 5 = | +0.571(e) | +0.143(e) | +0.286(e)= |  |
| 6 = | +0.571(f) | +0.143(f) | +0.286(f)= |  |

The output from the matrix is a set of location priority vectors that can now be assigned more qualitative project management categories. The higher the value means the higher the project management priority. Location features should be compared based on the following guideline in Table 22.

Table 22: Location Categorisation Guideline

| Location Priority Vector | Order of Location Feature Consideration |
| --- | --- |
| Highest | 1 |
|  | 2 |
|  | 3 |
|  | 4 |
|  | 5 |
| Lowest | 6 |

So essentially this demonstrates that the highest priority vector for a location should be given the first consideration, and so on until the lowest priority vector is reached.

## 12.2	Biodiesel Quality

It will also be crucial to maintain the quality of the biodiesel fuel that is sold, as this has very stringent regulations to avoid damaging vehicle engines. Fuel quality experts from: the European (ACEA); the US Association of International Automobile Manufacturers (AIAM); and the Japanese Automobile Manufacturers Association (JAMA); have developed the new biofuel guidelines for ethanol and biodiesel when blended with petrol and diesel. Fuel providers can follow the guidelines to maintain good biofuel quality anywhere from production to retail. Information on test methods and good fuel management practices are also given as part of the guidelines, which compliment the Worldwide Fuel Charter (WWFC) (Worldwide Fuel Charter, 2006).

It is important for all biodiesel producers to maintain these standards because mutual destruction rules are apparent. In this instance, if any provider sells unsuitable fuel to consumers and vehicles are damaged as a result, then consumers will likely be turned off from using biodiesel blends from all producers in the future no matter who the problem was with. For this reason self regulation may be an additional option for the industry players to implement.

## 12.3	UK Biodiesel Sales

Monthly UK biodiesel sales currently stand at more than 700,000 litres from more than 100 filling stations (Renewable Energy Association, 2009). The Renewable Transport Fuel Obligation (RTFO) in the UK requires all road transportation fuel suppliers to blend in 5 percent biofuel to all fuel that is supplied from the end of 2010 (UK Department for Transport, 2009). It is a very favourable system for biofuel producers, as it is implemented through a certification scheme governed by the Renewable Fuels Agency. This means companies who sell more than the 5% biofuel they are obliged to, are able to sell their excess certificates to those who failed to reach this target.

As has previously been discussed, the EU is planning to increase this minimum percentage in 2014 as well as the maximum blend percentage for biofuel, and therefore this will open up an even wider market for biodiesel (ACEA, 2008a). This will have the effect of increasing the upwards pressure on demand for biodiesel, and therefore also increase the price paid for biodiesel such that any algae biodiesel project will have increased revenues despite outputting the same amount.

In light of the information and analysis within this dissertation, it is clear to see that at this point in time, algae biodiesel production on a commercial scale is not economically feasible, however with the fast changing environment as well as the mounting investment in this area in terms of research and development, the technology is primed to become cost competitive with fossil fuel. In order for this to happen, any advances in the algae biofuel industry will have to reduce the cost of production of algae oil from around £2.00/L currently for a 10million L plant, to realistically less than £1.00/L. This is assuming that the retail margin required would be 40% and that the difference in breakeven per litre of biodiesel at the 10 million configuration would require a cost reduction per litre of £0.21 according to the NPV analysis in order to breakeven during the analysed time period.

## 12.4	Theoretical Contribution

A key theme that comes out of the findings, is that the struggle to make algae biodiesel profitable is not yet won, however is nearing completion. The exact cost pools that must be worked on to reduce cost further have been identified as well as the technical designs that could meet targets for future studies. A full through life cycle cost estimation has been achieved, compared to previous studies that either concentrated on a particular cost pool, or instead took a very arbitrary estimation methodology.

## 12.5	Future Areas for Research

Areas for future research that have been identified from the findings of this dissertation include:

•	Using a specialised algae biodiesel investor rate of return calculation instead of the method for calculating the required rate of return that is used in this study.

•	Performing a similar study for open/quasi closed systems in order to ascertain a scale barrier beyond which open systems may become more desirable than closed systems in terms of cost reduction through economies of scale.

•	Further research into the exact costs of algae biodiesel production based on the geographic region and its intrinsic government biofuel initiatives. This may direct algae biodiesel ventures to take place in certain countries in order to overcome the current higher price per litre, and act as technology incubation or test beds for further improvements that may make algae biodiesel profitable elsewhere.

# Conclusion

This dissertation aims to assess the technical and economic feasibility of algae biodiesel technology in its current state. Both of these aims have been met.

Algae Biodiesel is not better than current alternatives, however analysis indicates that it may be better than many future alternatives in terms of minimising cost, as well as protecting our environment and meeting emissions targets. Any new ventures in this area of the biofuels industry should therefore expect to need long term financial support before making any returns. In order to improve the likelihood of success it will be important to build great scale into the project, and for the price of crude oil to keep trending upwards. It is also apparent that any project will have to build in recycling processes for the excess by-products, as well as sales of the non-biodiesel products. In order to make a more effective and informed decision to proceed with a new algae biodiesel project, an update trial of productivities needs to be carried out, as well as an intermediate prototype scale plant.

Essentially, to become economically viable a litre of algae biodiesel must be producible at the same cost as the selling price of a litre of fossil diesel. For the purpose of this study, the long-run average of diesel per litre over the past ten years was taken in order to determine the viability price point, however it is important to note that the effects of the recession mean that as we move into the future, there will always be an upwards pressure on this average. To take account of this, a price band was calculated (pessimistic, optimistic, and most likely), rather than a price point. The desirable target will therefore be for algae biodiesel production costs per litre, to lie somewhere within this band. Over time the cost efficiency will increase and therefore should create the necessary profit margin for algae biodiesel companies, along with the effects of economies of scale, to make algae biodiesel attractive.

# References

AA, 2010. Fuel Price Reports, Petrol and Diesel Price Archive: 2000 to 2009, AA Fuel Reports 2000 through 2009.

Abadie L.M, Chamorro J.M, 2008. European CO2 Prices and Carbon Capture Investments, Energy Economics, vol 30, pg 2992–3015.

ACEA, 2008a. ACEA Statement on Biofuels.

ACEA, 2008b. EU Legislators Must Ensure Consumers Have Access to the Right Fuel for their Vehicles.

ACEA, 2009. Biofuels are Essential in Overall Approach to Reduce CO2 Emissions.

ACEA European Automobile Manufacturers’ Association, 2010. Diesel Market Highly Developed in Europe.

Anderson D, Masterson D, McDonald B, and Sullivan L, 2003. Industrial Biodiesel Plant Design and Engineering: Practical Experience, Crown Iron Works Company.

Anfavea, 2010. Autoveículos - Produção em 2010.

Barbosa M.J.G.V, 2003. Microalgal Photobioreactors: Scale-Up and Optimisation, Ph.D. Thesis, Wageningen University.

BBC, 2007. Quick guide: Biofuels, What are biofuels?

BBC News, 2010. BBC News Market Data: Brent Crude Oil Futures $/barrel.

Behzadi S, and Farid M.M, 2007. Review: Examining the Use of Different Feedstock for the Production of Biodiesel, Asia-Pacific Journal of Chemical Engineering, vol 2, 5, pg 480–486.

Bell E, and Bryman A, 2007. Ethical Codes and Management Research: A Comparative Content Analysis, British Journal of Management, 18, 1.

Bentley R.W, 2002. Global Oil and Gas Depletion: An Overview, Energy Policy, vol 30, pg 189–205.

Bot A.J, Nachtergaele F.O, and Young A, 2000. Land Resource Potential and Constraints at Regional and Country Levels, World Soil Resources Report.

Brent R.J, 1996. Applied Cost-Benefit Analysis, Edward Elgar Publishing Ltd.

Brock W, Lakonishok J, LeBaron J, 1992. Simple Technical Trading Rules and the Stochastic Properties of Stock Returns, The Journal of Finance, vol 47, 5, pg 1731-1764.

Brown P, 2009. Algal Biofuels Research, Development, and Commercialization Priorities: A Commercial Economics Perspective, Diversified Energy.

Bryman A, and Bell E, 2007. Business Research Methods, 2nd ed. Oxford: Oxford University Press.

Business Green, 2010. Senate Extends Controversial Biodiesel Tax Credit: Industry Urges Legislators to Move Bill to the President's Desk as Quickly as Possible.

Canakci M, and Gerpen J.V, 2001. Biodiesel Production from Oils and Fats with High Free Fatty Acids, American Society of Agricultural and Biological Engineers, Transactions of the ASABE, vol 44, 6, pg 1429–1436.

Carroll J, 2008. Petrobras Hires 80% of Deepwater Rigs, Inflates Rents (Update1), Bloomberg.

Cherry R.S, 2004. A Hydrogen Utopia?, National Academy of Engineering, International Journal of Hydrogen Energy, vol 29, pg 125-129.

Chisti Y, 2007. Biodiesel from Microalgae, Research Review Paper, Biotechnology Advances, vol 25, 3, pg 294-306.

Choi S-L, Suh I.S, Lee C-G, 2003. Lumostatic Operation of Bubble Column Photobioreactors for Haematococcus Pluvialis Cultures Using a Specific Light Uptake Rate as a Control Parameter, Enzyme and Microbial Technology, vol 33, pg 403–409.

Clean Tech Group LLC, 2009. Clean Technology Venture Investment Falls to $1 Billion in 1Q09.

Clymer F, 1955. Henry's Wonderful Model T, 1908-1927, McGraw-Hill: New York.

Crockett J, 2006. Feasibility Study for the Commercial Production of Biodiesel in the Magic Valley of Idaho, Idaho Department of Water Resources.

Department for Transport, 2008. Road Freight Statistics 2007, Transport Statistics Bulletin.

Dey P.K, 2002. Project Risk Management: A Combined Analytic Hierarchy Process and Decision Tree Approach, Cost Engineering, vol 44, 3.

Dickey P.A, 1959. The First Oil Well, Journal of Petroleum Technology, vol 11, 1, pg 14-26.

Dimitrov K, 2007. GreenFuel Technologies: A Case Study for Industrial Photosynthetic Energy Capture.

Dobbs T. L, 1988. Economic feasibility Methods: New Agricultural and Rural Enterprises (No. 703). Brookings, South Dakota State University.

Dufey A, 2006. Biofuels Production, Trade and Sustainable Development: Emerging Issues.

Edwards M.R, 2008. Green Algae Strategy: End Biowar I and Engineer Sustainable Food and Biofuels.

EFMA, 2007. ETS Will Have a Significant Impact on European Food Production, Annual Report (2008).

Elnashaie S, and Garhyan P, 2004. Bioethanol Production – Solving the Efficiency Bottleneck.

European Council for Automotive R&D (EUCAR), 2009. Collaborative Automotive R&D.

European Parliament and the EU Council, 2009. Setting Emission Performance Standards for New Passenger Cars as Part of the Community's Integrated Approach to Reduce CO2 Emissions from Light-Duty Vehicles. Legislative Acts and other Instruments. ENV 1046, ENT 338, CODEC 1910.

Eurostat, 2010b. Final Energy Consumption, by Sector - [tsdpc320]: Final energy consumption.

Eurostat, 2010c. Total Population - [tps00001]: At 1 January, European Statistics Database.

Falkowski P.G, and Raven J.A, 2007. Aquatic Photosynthesis, 2nd ed., Princeton.

Fattouh B, 2007. Analysing Oil Prices: The Usefulness and Limitations of Existing Approaches, Centre for Financial and Management Studies, SOAS, University of London & Oxford Institute for Energy Studies.

Fontaras G, Kousoulidou M, Karavalakis G, Tzamkiozis T, Pistikopoulos P, Ntziachristos L, Bakeas E, Stournas S, and Samaras Z, 2010. Effects of Low Concentration Biodiesel Blend Application on Modern Passenger Cars. Part 1: Feedstock Impact on Regulated Pollutants, Fuel Consumption and Particle Emissions, Environ Pollut, vol 158, 5, pg 1451-1460.

Gallagher B.J, 2010. The Economics of Producing Biodiesel from Algae, Journal of Renewable Energy, Elsevier.

Gerpen V, Pruszko R, Clements D, Shanks B, and Knothe G, 2005. Building a Successful Biodiesel Business, Manual Used in Biodiesel Workshops.

Ghirardi M.L, Togasaki R.K, and Seibert M, 1997. Oxygen Sensitivity of Algal H2- Production, Applied Biochemistry and Biotechnology, vol 63-65, 1, pg 141-151.

Global Bio-Energy Partnership, 2007. A Review of the Current State of Bioenergy Development in G8+5 Countries.

Grant A.T.J, 2003. A Model for Potential Cooperative Biodiesel Production in Gainesville, Alachua County, Florida, University of Florida Thesis.

Green Car Congress, 2010. Automakers in Brazil Hit 10M Flex-Fuel Vehicle Mark; Brazilian Sugarcane Association Urges Global Dissemination.

Greene D.L, Hopson J.L, and Li J, 2006. Have We Run Out of Oil Yet? Oil Peaking Analysis from an Optimist's Perspective, Energy Policy, vol 34, pg 515–531.

Haldi J, and Whitcomb D, 1967. Economies of Scale in Industrial Plants, The Journal of Political Economy, vol 75, 4, pg 373-385.

Hammersley M, 1996. The Relationship Between Qualitative Research: Paradigm Loyalty Versus Methodological Eclecticism, Richardson J.T.E Handbook of Research Methods for Psychology and the Social Sciences, Leicester: BPS Books.

Hopkin K, 2008. Using Crop Residue for Biofuels Hurts Soil Quality, Scientific American, Energy & Sustaniability.

Hosikian A, Lim S, Halim R, and Danquah M.K, 2010. Chlorophyll Extraction from Microalgae: A Review on the Process Engineering Aspects, International Journal of Chemical Engineering, vol 2010, Article ID 391632.

Humphreys K.K, 2004. Project and Cost Engineers' Handbook, 4th ed., Cost Estimating, CRC Press, Chapter 3, pg 57–86.

ICIS, 2010. Indicative Chemical Prices A-Z.

IEA, 2009. World Energy Outlook: Executive Summary, pg 8.

Kavalov B, and Peteves S.D, 2004. Impacts of the Increasing Automotive Diesel Consumption in the EU, European Comission, Directorate General Joint Research Centre, Document Number: EUR 21378 EN.

Kereliuka G.R, Sosulskib F.W, and Kaldy M.S, 1995. Carbohydrates of North American corn (Zea mays), Food Research International, vol 28, pg 311-315.

Kim S, and Dale B.E, 2004. Global Potential Bioethanol Production from Wasted Crops and Crop Residues, Biomass and Bioenergy, vol 26, pg 361-375.

Kizililsoley M, and Helvacioglu S, 2008. Micro-Algae Growth Technology Systems, Soley Institute.

Lee S-M, and Han H-S, 2008. Benefit-Cost Analysis of Biodiesel Production in Korea, Journal of Rural Development, vol 31, 5, pg 49-65.

Liansheng W, 2000. A Survey and an Analysis of Investors' Demands for Listed Companies' Accounting Information, Economic Research Journal.

Malakoff D, 1998. Coastal Ecology: Death by Suffocation in the Gulf of Mexico, Science (Washington), vol 281, 5374, pg 190-192.

Massey R, 2009. Motorists hit Again as Government Tax Grab Adds a Record 15p a Litre to the Price of Petrol, Mail Online.

Merino S.T, and Cherry J, 2007. Progress and Challenges in Enzyme Development for Biomass Utilization, Advances in Biochemical Engineering/Biotechnology, vol 108, pg 95-120.

Methanex, 2010. Products and Services: Methanol Price.

Methanol Institute and International Fuel Quality Center, 2006. A Biodiesel Primer: Market & Public Policy Developments, Quality, Standards & Handling, Arlington, VA: Methanol Institute.

Mirón A.S, Garcia M-C.C, Camacho F.G, Grima E.M, and Chisti Y, 2002. Growth and Biochemical Characterization of Microalgal Biomass Produced in Bubble Column and Airlift Photobioreactors: Studies in Fed-Batch Culture, Enzyme and Microbial Technology, vol 31, pg 1015–1023.

Molina E, Fernandez J, Acien F.G, and Chisti Y, 2001. Tubular Photobioreactor Design for Algal Cultures, Journal of Biotechnology, vol 92, pg 113–131.

Molina Grima E, Belarbi E.H, Acien Fernandez F.G, Robles Medina A, and Chisti Y, 2003. Recovery of Microalgal Biomass and Metabolites: Process Options and Economics, Biotechnology Advances, vol 20, pg 491–515.

Norman D. W, Worman F. D, Siebert J. D, and Modiakgotla E, 1995. The Farming Systems Approach to Development and Appropriate Technology Generation, Rome, Italy: Food and Agricultural Organization of the United Nations.

Pall Fuels and Chemicals, 2005. Field Applications HCP-4C: Refinery Filtration Applications.

Pickett J, Anderson D, Bowles D, Bridgwater T, Jarvis P, Mortimer N, Poliakoff M, Woods J, Heap R, Alam T, Edwards S, and Watson P, 2008. Sustainable Biofuels: Prospects and Challenges, The Royal Society, 4:33.

Pulz O, 2001. Photobioreactors: Production Systems for Phototrophic Microorganisms, Appl Microbiol Biotechnol, vol 57, pg 287–293.

Reece M, 2004. Energy Statistics Working Group Meeting, Special Issue Paper 9: Densities of Oil Products, IEA.

Reis N, 2006. Novel Oscillatory Flow Reactors for Biotechnological Applications, Chapter 3: The Novel Oscillatory Flow Reactor Designs, Ph.D. Thesis, University of Cambridge, Department of Chemical Engineering.

Reis N, 2008. Polymer Fluids: Other Projects and Collaborations, 2. Novel Oscillatory Flow Reactors for Biotechnological Applications (PhD work), University of Cambridge, Dept. Chemical Engineering.

Renewable Energy Association (REA), 2009. Biofuels; Biodiesel - Renewable Substitutes for Fossil-Sourced Diesel Fuel.

Romm, J, 2006. The Car and Fuel of the Future, Energy Policy, vol 34, pg 2609–2614.

Ross S.A, Westerfield R.W, Jaffe J, and Jordan B.D, 2009. Modern Financial Management, 8th ed., McGraw-Hill International Edition, Chap. 9.

Rubens C, 2008. WTF Are Fourth-Generation Biofuels?

Saaty T.L, 1980. The Analytic Hierarchy Process: Planning, Priority Setting, Resource Allocation,  	 McGraw-Hill International Book Co., New York and London.

Schubert C, 2006. Can Biofuels Finally Take Centre Stage?, Nature Biotechnology, vol 24, pg 777 – 784.

Sheehan J, Dunahay T, Benemann J, and Roessler P, 1998. A Look Back at the U.S. Department of Energy’s Aquatic Species Program—Biodiesel from Algae, National Renewable Energy Laboratory.

Standard and Poor’s Index Services, 2010. S&P 500 Monthly Returns.

Steger C, 2009. The Promise of Algae Biofuels - a New NRDC Report, NRDC Switchboard.

Takahama U, 1982. Suppression of Carotenoid Photobleaching by Kaempferol in Isolated Chloroplasts, Plant & Cell Physiol, vol 23, 5, pg 859-864.

Tilman D, Hill J, Lehman C, 2006. Carbon-Negative Biofuels from Low-Input High-Diversity Grassland Biomass, Science, vol 314, pg 1598-1600.

Tredici M.R, 1999. Bioreactors, Photo, In: Flickinger MC, Drew SW (eds) Encyclopedia of Bioprocess Technology: Fermentation, Biocatalysis and Bioseparation, vol 1, pg 395–419.

Tredici M.R, and Materassi R, 1992. From Open Ponds to Vertical Alveolar Panels: the Italian Experience in the Development of Reactors for the Mass Cultivation of Phototrophic Microorganisms, Journal of Applied Phycology, vol 4, 3, pg 221-231.

Trevino R, 2008. PreMBA Analytical Primer: Essential Quantitative Concepts for Business Math, Palgrave Macmillan.

Ugwu C.U, Aoyagi H, and Uchiyama H, 2008. Photobioreactors for Mass Cultivation of Algae, Bioresource Technology, vol 99, 10, pg 4021-4028.

Ugwu C.U, Ogbonna J.C, and Tanaka H, 2005. Light/Dark Cyclic Movement of Algal Culture (Synechocystis aquatilis ) in Outdoor Inclined Tubular Photobioreactor Equipped with Static Mixers for Efficient Production of Biomass, Biotechnology Letters, vol 27, 2, pg 75-78.

UK Department for Transport, 2009. Carbon and Sustainability Reporting within the Renewable Transport Fuel Obligation: Requirements and Guidance.

UK Department of Energy and Climate Change, 2009. Energy Statistics: Total Energy.

UN Department of Economic and Social Affairs, 2004. World Population to 2300, New York.

Vierhout R, 2010. Can Europe Solve the E85 Chicken-and-Egg Problem?, Ethanol Producer Magazine.

Whittington T, 2006. Biodiesel Production and Use by Farmers, is it Worth Considering?, Department of Agriculture and Food.

Whitton B.A, and Potts M, 2000. The Ecology of Cyanobacteria: Their Diversity in Time and Space, Springer, vol 2, 13.

Worldwide Fuel Charter, 2006. Worldwide Fuels Harmonisation, 4th ed.

X-rates, 2010. Great British Pounds to 1 USD.

You Y-D, Shie J-L, Chang C-Y, Huang S-H, Pai C-Y, Yu Y-H, and Chang C.H, 2008. Economic Cost Analysis of Biodiesel Production: Case in Soybean Oil, Energy Fuels, vol 22, 1, pg 182–189.

# Appendices

APPENDIX A: Calculations for UK Road Freight per person per day.

| UK Freight Weight /Tonnes/Annum | 1,869 million1 |
| --- | --- |
| Total UK Population | 607813522 |
| Freight Weight Per Capita /Tonnes/Annum | 30.75 |
| Freight Weight Per Capita /Kg/Aanum | 30749.56 |
| Freight Weight Per Capita /Kg/Day | 84.25 |

Data from: (1) Department for Transport, 2008. (2) Eurostat, 2010c.

APPENDIX B: Calculations for Potential Algae Yield Figure

| Feedstock Crop | Oil Yield / L/ha | Land Area Needed to satisfy 100% of European Transport Fuel Demand /Mha | Land Area Need as a % of Total Available Land Area in Europe |
| --- | --- | --- | --- |
| Corn | 172 | 2571931.40 | 708.29 |
| Soybean | 446 | 991865.92 | 273.15 |
| Canola | 1190 | 371741.34 | 102.37 |
| Jatropha | 1892 | 233811.95 | 64.39 |
| Coconut | 2689 | 164511.79 | 45.31 |
| Oil Palm | 5950 | 74348.27 | 20.47 |
| Microalgaea | 58700 | 7536.15 | 2.08 |
| Microalgaeb | 136900 | 3231.35 | 0.89 |
| a= 30% oil (by weight) in biomass |  |  |  |
| b= 70% oil (by weight) in biomass |  |  |  |
| Final Energy Consumption - EU Transport /1000 toe |  |  |  |
| 374269 | (Eurostat, 2010b) |  |  |
| Average Density of Crude Oil /g/cm3 |  |  |  |
| 0.84605 | (Reece, 2004) |  |  |
| Final Energy Consumption - EU Transport /1000 L |  |  |  |
| 442372200.2 | (mass=density*volume) |  |  |
| Total Suitable Land Area in Europe /Mha |  |  |  |
| 363120 | (Bot et al., 2000) |  |  |

(Oil Yield data from Chisti, 2007)

APPENDIX C: Table of Conversions used for Economic Viability Model Calculations

| Conversion Rates |  |  |
| --- | --- | --- |
| £1 = $1.6 | or | $1 = £0.625 |
| 1 EUR = 0.83 GBP |  |  |
| 1 Imperial gallon = 4.54609188 litres |  |  |
| 1 short ton = 907.18474 kg |  |  |
| 1 pound = 0.45359237 kilograms |  |  |
| Average Density of Crude Oil /g/cm3 |  |  |
| 0.84605 | (Reece, 2004) |  |
| 1 pound = 0.45359237 kg |  |  |
| 1 tonne = 1000 kg |  |  |

APPENDIX D: Raw Data for Retail Diesel Price

| Historic UK Diesel Retail Prices |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year | Month | Average Price £/L | Month | Rearragned for Median Price £/L |
| 2000 | Jan | 0.774 | Jan | 0.774 |
|  | Feb | 0.782 | Feb | 0.782 |
|  | Mar | 0.813 | Apr | 0.807 |
|  | Apr | 0.807 | Mar | 0.813 |
|  | May | 0.816 | Aug | 0.813 |
|  | Jun | 0.834 | May | 0.816 |
|  | Jul | 0.832 | Oct | 0.829 |
|  | Aug | 0.813 | Jul | 0.832 |
|  | Sep | 0.842 | Jun | 0.834 |
|  | Oct | 0.829 | Sep | 0.842 |
|  | Nov | 0.851 | Nov | 0.851 |
|  | Dec | 0.851 | Dec | 0.851 |
| 2001 | Jan | 0.824 | Dec | 0.754 |
|  | Feb | 0.822 | Nov | 0.762 |
|  | Mar | 0.785 | Oct | 0.773 |
|  | Apr | 0.785 | Aug | 0.782 |
|  | May | 0.789 | Sep | 0.783 |
|  | Jun | 0.790 | Mar | 0.785 |
|  | Jul | 0.785 | Apr | 0.785 |
|  | Aug | 0.782 | Jul | 0.785 |
|  | Sep | 0.783 | May | 0.789 |
|  | Oct | 0.773 | Jun | 0.790 |
|  | Nov | 0.762 | Feb | 0.822 |
|  | Dec | 0.754 | Jan | 0.824 |
| 2002 | Jan | 0.754 | Feb | 0.753 |
|  | Feb | 0.753 | Jan | 0.754 |
|  | Mar | 0.759 | Mar | 0.759 |
|  | Apr | 0.773 | Aug | 0.760 |
|  | May | 0.768 | Jul | 0.761 |
|  | Jun | 0.763 | Dec | 0.761 |
|  | Jul | 0.761 | Nov | 0.762 |
|  | Aug | 0.760 | Jun | 0.763 |
|  | Sep | 0.767 | Oct | 0.766 |
|  | Oct | 0.766 | Sep | 0.767 |
|  | Nov | 0.762 | May | 0.768 |
|  | Dec | 0.761 | Apr | 0.773 |
| 2003 | Jan | 0.775 | Jul | 0.770 |
|  | Feb | 0.781 | Jun | 0.771 |
|  | Mar | 0.815 | Jan | 0.775 |
|  | Apr | 0.810 | Aug | 0.778 |
|  | May | 0.780 | Sep | 0.779 |
|  | Jun | 0.771 | May | 0.780 |
|  | Jul | 0.770 | Oct | 0.780 |
|  | Aug | 0.778 | Feb | 0.781 |
|  | Sep | 0.779 | Nov | 0.781 |
|  | Oct | 0.780 | Dec | 0.782 |
|  | Nov | 0.781 | Apr | 0.810 |
|  | Dec | 0.782 | Mar | 0.815 |
| 2004 | Jan | 0.785 | Jan | 0.785 |
|  | Feb | 0.786 | Feb | 0.786 |
|  | Mar | 0.793 | Mar | 0.793 |
|  | Apr | 0.799 | Apr | 0.799 |
|  | May | 0.828 | Jul | 0.820 |
|  | Jun | 0.828 | May | 0.828 |
|  | Jul | 0.820 | Jun | 0.828 |
|  | Aug | 0.828 | Aug | 0.828 |
|  | Sep | 0.835 | Sep | 0.835 |
|  | Oct | 0.861 | Oct | 0.861 |
|  | Nov | 0.870 | Dec | 0.861 |
|  | Dec | 0.861 | Nov | 0.870 |
| 2005 | Jan | 0.848 | Feb | 0.832 |
|  | Feb | 0.832 | Mar | 0.846 |
|  | Mar | 0.846 | Jan | 0.848 |
|  | Apr | 0.887 | Jun | 0.879 |
|  | May | 0.883 | May | 0.883 |
|  | Jun | 0.879 | Apr | 0.887 |
|  | Jul | 0.911 | Jul | 0.911 |
|  | Aug | 0.935 | Dec | 0.923 |
|  | Sep | 0.953 | Aug | 0.935 |
|  | Oct | 0.959 | Nov | 0.952 |
|  | Nov | 0.952 | Sep | 0.953 |
|  | Dec | 0.923 | Oct | 0.959 |
| 2006 | Jan | 0.935 | Nov | 0.916 |
|  | Feb | 0.941 | Oct | 0.921 |
|  | Mar | 0.943 | Jan | 0.935 |
|  | Apr | 0.978 | Dec | 0.937 |
|  | May | 0.990 | Feb | 0.941 |
|  | Jun | 0.981 | Mar | 0.943 |
|  | Jul | 0.991 | Sep | 0.950 |
|  | Aug | 0.995 | Apr | 0.978 |
|  | Sep | 0.950 | Jun | 0.981 |
|  | Oct | 0.921 | May | 0.990 |
|  | Nov | 0.916 | Jul | 0.991 |
|  | Dec | 0.937 | Aug | 0.995 |
| 2007 | Jan | 0.921 | Feb | 0.909 |
|  | Feb | 0.909 | Jan | 0.921 |
|  | Mar | 0.927 | Mar | 0.927 |
|  | Apr | 0.951 | Apr | 0.951 |
|  | May | 0.968 | May | 0.968 |
|  | Jun | 0.974 | Sep | 0.969 |
|  | Jul | 0.972 | Aug | 0.970 |
|  | Aug | 0.970 | Jul | 0.972 |
|  | Sep | 0.969 | Jun | 0.974 |
|  | Oct | 0.998 | Oct | 0.998 |
|  | Nov | 1.054 | Nov | 1.054 |
|  | Dec | 1.080 | Dec | 1.080 |
| 2008 | Jan | 1.092 | Dec | 1.019 |
|  | Feb | 1.093 | Nov | 1.088 |
|  | Mar | 1.143 | Jan | 1.092 |
|  | Apr | 1.174 | Feb | 1.093 |
|  | May | 1.242 | Mar | 1.143 |
|  | Jun | 1.316 | Apr | 1.174 |
|  | Jul | 1.331 | Oct | 1.177 |
|  | Aug | 1.256 | May | 1.242 |
|  | Sep | 1.243 | Sep | 1.243 |
|  | Oct | 1.177 | Aug | 1.256 |
|  | Nov | 1.088 | Jun | 1.316 |
|  | Dec | 1.019 | Jul | 1.331 |
| 2009 | Jan | 0.987 | Jan | 0.987 |
|  | Feb | 1.008 | Mar | 0.998 |
|  | Mar | 0.998 | Feb | 1.008 |
|  | Apr | 1.027 | Apr | 1.027 |
|  | May | 1.035 | May | 1.035 |
|  | Jun | 1.048 | Jul | 1.041 |
|  | Jul | 1.041 | Jun | 1.048 |
|  | Aug | 1.050 | Aug | 1.050 |
|  | Sep | 1.071 | Oct | 1.062 |
|  | Oct | 1.062 | Sep | 1.071 |
|  | Nov | 1.100 | Dec | 1.099 |
|  | Dec | 1.099 | Nov | 1.100 |

APPENDIX E: Standard and Poor’s Historical Market Returns Data

| Returns Series |  |  |  |
| --- | --- | --- | --- |
| Time Period | S&P 500 returns | ∆ Market Return (X) | ∆Diesel Price Return (Y) |
| Jan-00 | 1394.46 |  |  |
| Feb-00 | 1366.42 | -0.020 | 0.010 |
| Mar-00 | 1498.58 | 0.097 | 0.040 |
| Apr-00 | 1452.43 | -0.031 | -0.007 |
| May-00 | 1420.60 | -0.022 | 0.011 |
| Jun-00 | 1454.60 | 0.024 | 0.022 |
| Jul-00 | 1430.83 | -0.016 | -0.002 |
| Aug-00 | 1517.68 | 0.061 | -0.023 |
| Sep-00 | 1436.51 | -0.053 | 0.036 |
| Oct-00 | 1429.40 | -0.005 | -0.015 |
| Nov-00 | 1314.95 | -0.080 | 0.027 |
| Dec-00 | 1320.28 | 0.004 | 0.000 |
| Jan-01 | 1366.01 | 0.035 | -0.032 |
| Feb-01 | 1239.94 | -0.092 | -0.002 |
| Mar-01 | 1160.33 | -0.064 | -0.045 |
| Apr-01 | 1249.46 | 0.077 | 0.000 |
| May-01 | 1255.82 | 0.005 | 0.005 |
| Jun-01 | 1224.42 | -0.025 | 0.001 |
| Jul-01 | 1211.23 | -0.011 | -0.006 |
| Aug-01 | 1133.58 | -0.064 | -0.004 |
| Sep-01 | 1040.94 | -0.082 | 0.001 |
| Oct-01 | 1059.78 | 0.018 | -0.013 |
| Nov-01 | 1139.45 | 0.075 | -0.014 |
| Dec-01 | 1148.08 | 0.008 | -0.010 |
| Jan-02 | 1130.20 | -0.016 | 0.000 |
| Feb-02 | 1106.73 | -0.021 | -0.001 |
| Mar-02 | 1147.39 | 0.037 | 0.008 |
| Apr-02 | 1076.92 | -0.061 | 0.018 |
| May-02 | 1067.14 | -0.009 | -0.006 |
| Jun-02 | 989.81 | -0.072 | -0.007 |
| Jul-02 | 911.62 | -0.079 | -0.003 |
| Aug-02 | 916.07 | 0.005 | -0.001 |
| Sep-02 | 815.28 | -0.110 | 0.009 |
| Oct-02 | 885.76 | 0.086 | -0.001 |
| Nov-02 | 936.31 | 0.057 | -0.005 |
| Dec-02 | 879.82 | -0.060 | -0.001 |
| Jan-03 | 855.70 | -0.027 | 0.018 |
| Feb-03 | 841.15 | -0.017 | 0.008 |
| Mar-03 | 848.18 | 0.008 | 0.044 |
| Apr-03 | 916.92 | 0.081 | -0.006 |
| May-03 | 963.59 | 0.051 | -0.037 |
| Jun-03 | 974.50 | 0.011 | -0.012 |
| Jul-03 | 990.31 | 0.016 | -0.001 |
| Aug-03 | 1008.01 | 0.018 | 0.010 |
| Sep-03 | 995.97 | -0.012 | 0.001 |
| Oct-03 | 1050.71 | 0.055 | 0.001 |
| Nov-03 | 1058.20 | 0.007 | 0.001 |
| Dec-03 | 1111.92 | 0.051 | 0.001 |
| Jan-04 | 1131.13 | 0.017 | 0.004 |
| Feb-04 | 1144.94 | 0.012 | 0.001 |
| Mar-04 | 1126.21 | -0.016 | 0.009 |
| Apr-04 | 1107.30 | -0.017 | 0.008 |
| May-04 | 1120.68 | 0.012 | 0.036 |
| Jun-04 | 1140.84 | 0.018 | 0.000 |
| Jul-04 | 1101.72 | -0.034 | -0.010 |
| Aug-04 | 1104.24 | 0.002 | 0.010 |
| Sep-04 | 1114.58 | 0.009 | 0.008 |
| Oct-04 | 1130.20 | 0.014 | 0.031 |
| Nov-04 | 1173.82 | 0.039 | 0.010 |
| Dec-04 | 1211.92 | 0.032 | -0.010 |
| Jan-05 | 1181.27 | -0.025 | -0.015 |
| Feb-05 | 1203.60 | 0.019 | -0.019 |
| Mar-05 | 1180.59 | -0.019 | 0.017 |
| Apr-05 | 1156.85 | -0.020 | 0.048 |
| May-05 | 1191.50 | 0.030 | -0.005 |
| Jun-05 | 1191.33 | 0.000 | -0.005 |
| Jul-05 | 1234.18 | 0.036 | 0.036 |
| Aug-05 | 1220.33 | -0.011 | 0.026 |
| Sep-05 | 1228.81 | 0.007 | 0.019 |
| Oct-05 | 1207.01 | -0.018 | 0.006 |
| Nov-05 | 1249.48 | 0.035 | -0.007 |
| Dec-05 | 1248.29 | -0.001 | -0.030 |
| Jan-06 | 1280.08 | 0.025 | 0.013 |
| Feb-06 | 1280.66 | 0.000 | 0.006 |
| Mar-06 | 1294.83 | 0.011 | 0.002 |
| Apr-06 | 1310.61 | 0.012 | 0.037 |
| May-06 | 1270.09 | -0.031 | 0.012 |
| Jun-06 | 1270.20 | 0.000 | -0.009 |
| Jul-06 | 1276.66 | 0.005 | 0.010 |
| Aug-06 | 1303.82 | 0.021 | 0.004 |
| Sep-06 | 1335.85 | 0.025 | -0.045 |
| Oct-06 | 1377.94 | 0.032 | -0.031 |
| Nov-06 | 1400.63 | 0.016 | -0.005 |
| Dec-06 | 1418.30 | 0.013 | 0.023 |
| Jan-07 | 1438.24 | 0.014 | -0.017 |
| Feb-07 | 1406.82 | -0.022 | -0.013 |
| Mar-07 | 1420.86 | 0.010 | 0.020 |
| Apr-07 | 1482.37 | 0.043 | 0.026 |
| May-07 | 1530.62 | 0.033 | 0.018 |
| Jun-07 | 1503.35 | -0.018 | 0.006 |
| Jul-07 | 1455.27 | -0.032 | -0.002 |
| Aug-07 | 1473.99 | 0.013 | -0.002 |
| Sep-07 | 1526.75 | 0.036 | -0.001 |
| Oct-07 | 1549.38 | 0.015 | 0.030 |
| Nov-07 | 1481.14 | -0.044 | 0.056 |
| Dec-07 | 1468.36 | -0.009 | 0.025 |
| Jan-08 | 1378.55 | -0.061 | 0.011 |
| Feb-08 | 1330.63 | -0.035 | 0.001 |
| Mar-08 | 1322.70 | -0.006 | 0.046 |
| Apr-08 | 1385.59 | 0.048 | 0.027 |
| May-08 | 1400.38 | 0.011 | 0.058 |
| Jun-08 | 1280.00 | -0.086 | 0.060 |
| Jul-08 | 1267.38 | -0.010 | 0.011 |
| Aug-08 | 1282.83 | 0.012 | -0.056 |
| Sep-08 | 1166.36 | -0.091 | -0.010 |
| Oct-08 | 968.75 | -0.169 | -0.053 |
| Nov-08 | 896.24 | -0.075 | -0.076 |
| Dec-08 | 903.25 | 0.008 | -0.063 |
| Jan-09 | 825.88 | -0.086 | -0.031 |
| Feb-09 | 735.09 | -0.110 | 0.021 |
| Mar-09 | 797.87 | 0.085 | -0.010 |
| Apr-09 | 872.81 | 0.094 | 0.029 |
| May-09 | 919.14 | 0.053 | 0.008 |
| Jun-09 | 919.32 | 0.000 | 0.013 |
| Jul-09 | 987.48 | 0.074 | -0.007 |
| Aug-09 | 1020.62 | 0.034 | 0.009 |
| Sep-09 | 1057.08 | 0.036 | 0.020 |
| Oct-09 | 1036.19 | -0.020 | -0.008 |
| Nov-09 |  |  |  |
| Dec-09 |  |  |  |

APPENDIX F: Net Present Value Results

| NPV Analysis for 100,000L Plant Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | c0 /£ | 386263.376 |  |  |  |  |  |  |  |  |  |  |
|  |  | r | 0.0408985 |  |  |  |  |  |  |  |  |  |  |
| Year | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 | 2020 | 2021 | 2022 | 2023 |
| time period (t) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| Diesel Price Forecast |  | £1.048 | £1.072 | £1.097 | £1.121 | £1.146 | £1.170 | £1.195 | £1.219 | £1.244 | £1.268 | £1.292 | £1.317 |
| 1. Biodiesel Sales Revenue (less VAT 20%) |  | £83,840 | £85,760 | £87,760 | £89,680 | £91,680 | £93,600 | £95,600 | £97,520 | £99,520 | £101,440 | £103,360 | £105,360 |
| C1 (1. less total production costs) |  | -£93,659 | -£91,739 | -£89,739 | -£87,819 | -£85,819 | -£83,899 | -£81,899 | -£79,979 | -£77,979 | -£76,059 | -£74,139 | -£72,139 |
| Discount factor (1-r)^t |  | 1.041 | 1.083 | 1.128 | 1.174 | 1.222 | 1.272 | 1.324 | 1.378 | 1.434 | 1.493 | 1.554 | 1.618 |
| Discounted Cashflow | -£386,263 | -£89,979 | -£84,672 | -£79,571 | -£74,809 | -£70,233 | -£65,964 | -£61,862 | -£58,038 | -£54,363 | -£50,941 | -£47,704 | -£44,593 |
| Net Present Value | -£1,168,992 |  |  |  |  |  |  |  |  |  |  |  |  |
| Payback Time | Never |  |  |  |  |  |  |  |  |  |  |  |  |

| NPV Analysis for 10,000,000L Plant Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | c0 /£ | 9179686.57 |  |  |  |  |  |  |  |  |  |  |
|  |  | r | 0.0408985 |  |  |  |  |  |  |  |  |  |  |
| Year | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 | 2020 | 2021 | 2022 | 2023 |
| time period (t) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| Diesel Price Forecast |  | £1.048 | £1.072 | £1.097 | £1.121 | £1.146 | £1.170 | £1.195 | £1.219 | £1.244 | £1.268 | £1.292 | £1.317 |
| 1. Biodiesel Sales Revenue (less VAT 20%) |  | £8,384,000 | £8,576,000 | £8,776,000 | £8,968,000 | £9,168,000 | £9,360,000 | £9,560,000 | £9,752,000 | £9,952,000 | £10,144,000 | £10,336,000 | £10,536,000 |
| C1 (1. less total production costs) |  | -£2,808,786 | -£2,616,786 | -£2,416,786 | -£2,224,786 | -£2,024,786 | -£1,832,786 | -£1,632,786 | -£1,440,786 | -£1,240,786 | -£1,048,786 | -£856,786 | -£656,786 |
| Discount factor (1-r)^t |  | 1.041 | 1.083 | 1.128 | 1.174 | 1.222 | 1.272 | 1.324 | 1.378 | 1.434 | 1.493 | 1.554 | 1.618 |
| Discounted Cashflow | -£9,179,687 | -£2,698,424 | -£2,415,190 | -£2,142,955 | -£1,895,198 | -£1,657,056 | -£1,440,991 | -£1,233,305 | -£1,045,520 | -£865,010 | -£702,430 | -£551,290 | -£405,997 |
| Net Present Value | -£26,233,054 |  |  |  |  |  |  |  |  |  |  |  |  |
| Payback Time | Never |  |  |  |  |  |  |  |  |  |  |  |  |

APPENDIX G: Cost Pool Breakdowns

| 100,000 | L | Costs |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cost Pool | Description | Fixed Capital | Fixed Annual | Total Cost per Litre Biodiesel /£ | % of total cost |
| 1 | Algae biomass porduction | 144104.8402 | 40270.15981 | 1.84375 | 0.317271454 |
| 2 | Algae oil extraction | 241172.2559 | 153535.1527 | 3.947074086 | 0.67921027 |
| 3 | Transesterification (biodiesel production) | 986.2798135 | 1058.285025 | 0.020445648 | 0.003518276 |
| Total |  | 386263.3759 | 194863.5975 | 5.811269734 |  |

| 10,000,000 | L | Costs |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cost Pool | Description | Fixed Capital | Fixed Annual | Total Cost per Litre Biodiesel /£ | % of total cost |
| 1 | Algae biomass porduction | 2295907.623 | 641592.3766 | 0.29375 | 0.142257831 |
| 2 | Algae oil extraction | 6856489.156 | 10722019.37 | 1.757850852 | 0.85129549 |
| 3 | Transesterification (biodiesel production) | 27289.7869 | 105828.5025 | 0.013311829 | 0.006446679 |
| Total |  | 9179686.566 | 11469440.24 | 2.064912681 |  |